- Arc Hydro Tools Overview

Version 1.0 Beta 2, May 2002

Copyright © 2002 ESRI

All rights reserved.

Printed in the United States of America.

The information contained in this document is the exclusive property of ESRI.  This work is protected under United States copyright law and other international copyright treaties and conventions.  No part of this work may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, or by any information storage or retrieval system, except as expressly permitted in writing by ESRI. All requests should be sent to Attention: Contracts Manager, ESRI, 380 New York Street, Redlands, CA 92373-8100, USA.

The information contained in this document is subject to change without notice.

U.S. GOVERNMENT RESTRICTED/LIMITED RIGHTS

Any software, documentation, and/or data delivered hereunder is subject to the terms of the License Agreement.  In no event shall the U.S. Government acquire greater than RESTRICTED/LIMITED RIGHTS.  At a minimum, use, duplication, or disclosure by the U.S. Government is subject to restrictions as set forth in FAR §52.227-14 Alternates I, II, and III (JUN 1987); FAR §52.227-19 (JUN 1987) and/or FAR §12.211/12.212 (Commercial Technical Data/Computer Software); and DFARS §252.227-7015 (NOV 1995) (Technical Data) and/or DFARS §227.7202 (Computer Software), as applicable.  Contractor/Manufacturer is ESRI, 380 New York Street, Redlands, CA 92373-8100, USA.

ESRI, ARC/INFO, ArcCAD, ArcIMS, ArcView, BusinessMAP, MapObjects, PC ARC/INFO, SDE, and the ESRI globe logo are trademarks of ESRI, registered in the United States and certain other countries; registration is pending in the European Community. 3D Analyst, ADF, the ARC/INFO logo, AML, ArcNews, ArcTIN, the ArcTIN logo, ArcCOGO, the ArcCOGO logo, ArcGrid, the ArcGrid logo, ArcInfo, the ArcInfo logo, ArcInfo Librarian,  ArcInfo—Professional GIS, ArcInfo—The World's GIS, ArcAtlas, the ArcAtlas logo, the ArcCAD logo, the ArcCAD WorkBench logo, ArcCatalog, the ArcData logo, the ArcData Online logo, ArcDoc, ArcEdit, the ArcEdit logo, ArcEditor, ArcEurope, the ArcEurope logo, ArcExplorer, the ArcExplorer logo, ArcExpress, the ArcExpress logo, ArcFM, the ArcFM logo, the ArcFM Viewer logo, ArcGIS, the ArcGIS logo, the ArcIMS logo, ArcNetwork, the ArcNetwork logo, ArcLogistics, the ArcLogistics Route logo, ArcMap, ArcObjects, ArcPad, the ArcPad logo, ArcPlot, the ArcPlot logo, ArcPress, the ArcPress logo, the ArcPress for ArcView logo, ArcReader, ArcScan, the ArcScan logo, ArcScene, the ArcScene logo, ArcSchool, ArcSDE, the ArcSDE logo, the ArcSDE CAD Client logo, ArcSdl, ArcStorm, the ArcStorm logo, ArcSurvey, ArcToolbox, ArcTools, the ArcTools logo, ArcUSA, the ArcUSA logo, ArcUser, the ArcView logo, the ArcView GIS logo, the ArcView 3D Analyst logo, the ArcView Business Analyst logo, the ArcView Data Publisher logo, the ArcView Image Analysis logo, the ArcView Internet Map Server logo, the ArcView Network Analyst logo, the ArcView Spatial Analyst logo, the ArcView StreetMap logo, the ArcView StreetMap 2000 logo, the ArcView Tracking Analyst logo, ArcVoyager, ArcWorld, the ArcWorld logo, Atlas GIS, the Atlas GIS logo, AtlasWare, Avenue, the Avenue logo, the BusinessMAP logo, the Data Automation Kit logo, Database Integrator, DBI Kit, the Digital Chart of the World logo, the ESRI Data logo, the ESRI Press logo, ESRI—Team GIS, ESRI—The GIS People, FormEdit, Geographic Design System, Geography Matters, GIS by ESRI, GIS Day, GIS for Everyone, GISData Server, InsiteMAP, MapBeans, MapCafé, the MapCafé logo, the MapObjects logo, the MapObjects Internet Map Server logo, ModelBuilder, MOLE, the MOLE logo, NetEngine, the NetEngine logo, the PC ARC/INFO logo, PC ARCEDIT, PC ARCPLOT, PC ARCSHELL, PC DATA CONVERSION, PC NETWORK, PC OVERLAY, PC STARTER KIT, PC TABLES, the Production Line Tool Set logo, RouteMAP, the RouteMAP logo, the RouteMAP IMS logo, Spatial Database Engine, the SDE logo, SML, StreetEditor, StreetMap, TABLES, The World's Leading Desktop GIS, Water Writes, and Your Personal Geographic Information System are trademarks; and ArcData, ArcOpen, ArcQuest, ArcWatch, ArcWeb, Rent-a-Tech, Geography Network, the Geography Network logo, www.geographynetwork.com, www.gisday.com, @esri.com, and www.esri.com are service marks of ESRI.

Other companies and products mentioned herein are trademarks or registered trademarks of their respective trademark owners.

Table of Contents

Introduction to Arc Hydro	1

Introduction to Arc Hydro Tools	1

Tool list	2

Terrain Preprocessing	2

Watershed Processing	3

Attribute Tools	3

Network Tools	4

Buttons	5

## Introduction to Arc Hydro

Arc Hydro is an ArcGIS-based system geared to support water resources applications.  It consists of two key components:

- Arc Hydro Data ModelAbout_ArcHydro_Data_Model

- Arc Hydro ToolsAbout_ArcHydro_Tools

These two components, together with the generic programming framework, provide basic database design and set of tools that facilitate analyses often performed in the water resources area.  Arc Hydro is intended to provide the initial functionality that can then be expanded by adding to it database structures and functions required by a specific task or application.

## Introduction to Arc Hydro Tools

The Arc Hydro tools are a set of utilities developed on top of the Arc Hydro data model.  They operate in the ArcGIS environment.  Some of the functions require the Spatial Analyst extension.  The tools are accessed through the Arc Hydro Tools toolbar, where they are grouped by functionality into five menus and six buttons.

- Menus:

  - Terrain Preprocessing.  Tools in this menu deal with processing of Digital Elevation Model (DEM).  They are mostly used once in order to prepare spatial information for later use.

  - Watershed Processing.  Tools in this menu deal with watershed and subwatershed delineation and basin characteristic determination.  They operate on top of the spatial data prepared in the terrain preprocessing stage.

  - Attribute Tools.  These tools provide functionality for generation of some of the key attributes (fields) in the Arc Hydro data model.  Some of the tools require existence of a geometric network.

  - Network tools.  These tools generate or manipulate properties of geometric (hydro) network.

  - ApUtilities.  These are tools for management of Arc Hydro project properties.  In general, they will be seldom used.

- Buttons:

  - Flow Path Tracing

  - Point Delineation

  - Batch Point Generation

  - Assign Related Identifier

  - Global Delineation

  - Trace By NextDownID Attribute

The tools have two key purposes.  The first purpose is to manipulate (assign) key attributes in the Arc Hydro data model.  These attributes form the basis for further analyses.  They include the key identifiers (such as HydroID, DrainID, NextDownID, etc.) and the measure attributes (such as LengthDown).  The second purpose for the tools is to provide some core functionality often used in water resources applications.  This includes DEM-based watershed delineation, network generation, and attribute-based tracing.

The functionality of Arc Hydro tools is expected to grow over time.  They have been implemented in a way that allows easy addition to their functionality, either internally (by adding additional code) or externally, by providing additional functionality through the use of key Arc Hydro data structures.

## Tool list

The tools will be presented by their grouping in the user interface (menus).  The ApUtilities tools will not be addressed here, as they are general-purpose utilities not related to the functionality of Arc Hydro tools.

### Terrain Preprocessing

| Tool | Description |
| --- | --- |
| DEM Reconditioning | Enforce linear drainage pattern (vector) onto a DEM (grid). Implements AGREE methodology. |
| Fill Sinks | Fill sinks for an entire DEM (grid). |
| Flow Direction | Create flow direction grid from a DEM grid. |
| Flow Accumulation | Create flow accumulation grid from a flow direction grid. |
| Stream Definition | Create a new grid (stream grid) with cells from a flow accumulation grid that exceed used-defined threshold. |
| Stream Segmentation | Create a stream link grid from the stream grid (every link between two stream junction gets a unique identifier). |
| Catchment Grid Delineation | Create a catchment grid for segments in the stream link grid. It identifies areas draining into each stream link. |
| Catchment Polygon Processing | Create catchment polygons out of the catchment grid. |
| Drainage Line Processing | Create streamlines out of the stream link grid. |
| Adjoint Catchment Processing | Create adjoint catchment polygon for each catchment in the catchment polygon feature class. Adjoint catchment is total upstream area (if any) draining into a single catchment. |
| Drainage Point Processing | Create a drainage point at the most downstream point in the catchment (center of a grid cell with the largest value in the flow accumulation grid for that catchment). |

### Watershed Processing

| Tool | Description |
| --- | --- |
| Batch Watershed Processing | Create a watershed for every point in the batch point feature class. Results are stored in a watershed (polygon) feature class. Watersheds are overlapping (if points are on the same stream). |
| Batch Subwatershed Processing | Create a subwatershed for every point in the batch point feature class. Results are stored in a subwatershed (polygon) feature class. Subwatersheds are non-overlapping (if points are on the same stream). |
| Drainage Area Centroid | Create a point at the centroid of each polygon in a polygon feature class and store it in a point feature class. |
| Longest Flow Path | Create a line following the longest flow path based on the steepest descent (as defined by the flow direction grid) in a catchment or watershed. |

### Attribute Tools

| Tool | Description |
| --- | --- |
| Assign HydroID | Assigns a unique identifier (HydroID) to a feature. HydroID is unique across a geodatabase. |
| Generate From/To Node for Lines | Generates from-node/to-node topology based on physical line connectivity for a line feature class (does not require hydro network). Nodes are defined as ends of lines. They are not created as a separate feature class, but rather just identified and accounted for internally. |
| Find Next Downstream Line | Find the HydroID of the next downstream linear feature class and store it in the NextDownID field of the feature. The directionality is based on the digitized direction. Connectivity is established by the physical connection of the linear features (does not require hydro network). |
| Calculate Length Downstream for Edges | Calculate length from the downstream end of a hydro edge to the outlet of the hydro network (requires hydro network). The length is stored in the LengthDown field. |
| Calculate Length Downstream for Junctions | Calculate length from a hydro junction to the outlet of the hydro network (requires hydro network). The length is stored in the LengthDown field. |
| Find Next Downstream Junction | Find the HydroID of the next downstream junction and store it in the NextDownID field of the junction feature (requires hydro network). |
| Store Area Outlets | Identify most likely hydro junction that drains an area. The HydroID of that junction is stored in the JunctionID field for the area feature class. |
| Consolidate Attributes | Summarize the values of a numerical attribute of a feature class and store them in a field in another (or same) feature class. Relationship between the from and the to feature class is established through related IDs. Operators include sum, min, max, average, median, mode, standard deviation, and count. User specifies the from and the to feature classes, what field to summarize and in what field to store the summarized values. The tool can use the same feature class as both from and to objects to operate on. |
| Accumulate Attributes | Summarize the values of a numerical attribute of a feature class and store them in a field in another (or same) feature class. The tool selects the upstream objects by tracing either using the geometric network or a NextDownID relationship, and summarizes the selected objects. Operators include sum, min, max, average, median, mode, standard deviation, and count. The selectable objects are either the traceable objects, or can be in an ID-related feature class (using existing relationship classes). User specifies the from and the to feature classes, what field to summarize, and in what field to store the summarized values |

### Network Tools

| Tool | Description |
| --- | --- |
| Hydro Network Generation | Generate a hydro network (hydro edges and hydro junctions) from drainage lines, catchments, and drainage points. The function updates all the connectivity fields in input feature classes. |
| Node/Link Schema Generation | Generate schematic (node-link) network by connecting centers of catchments/drainage areas and junctions, and junctions and junctions. Connectivity is established through connectivity fields (attributes), not physical connectivity. |
| Store Flow Direction | Store information about hydro (geometric) network element’s directionality into an attribute of the feature matching the element. |
| Set Flow Direction | Define flow direction for a geometric network based on digitized direction or an attribute for the feature. |

### Buttons

| Tool | Description |
| --- | --- |
| Flow Path Tracing | Trace the downstream path, based on the steepest descent, from a user specified point to the edge of the DEM (uses flow direction grid). |
| Point Delineation | Delineate a watershed for a user specified point (interactive) based on the preprocessed DEM. |
| Batch Point Generation | Add user specified point to a batch point feature class. This point feature class can be used as an input to the batch watershed and subwatershed delineation functions. |
| Assign Related Identifier | Interactively assign a value of a field in a source feature to a field in the target feature. User specifies both the source and target feature classes and fields. |
| Global Point Delineation | Delineate a watershed for a user specified point (interactive) based on a set of preprocessed geographic units tied together by a geometric network. |
| Trace By NextDownID Attribute | Using the attribute relationship established through NextDownID field, trace from a selected location upstream, downstream, or in both directions. The final selected features can include the objects selected through the trace, and/or ID-related objects (using existing relationship classes). |