Online Help for Arc Hydro Tools

Version 1.0 Beta 2, May 2002

Copyright © 2002 ESRI

All rights reserved.

Printed in the United States of America.

The information contained in this document is the exclusive property of ESRI.  This work is protected under United States copyright law and other international copyright treaties and conventions.  No part of this work may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, or by any information storage or retrieval system, except as expressly permitted in writing by ESRI. All requests should be sent to Attention: Contracts Manager, ESRI, 380 New York Street, Redlands, CA 92373-8100, USA.

The information contained in this document is subject to change without notice.

U.S. GOVERNMENT RESTRICTED/LIMITED RIGHTS

Any software, documentation, and/or data delivered hereunder is subject to the terms of the License Agreement.  In no event shall the U.S. Government acquire greater than RESTRICTED/LIMITED RIGHTS.  At a minimum, use, duplication, or disclosure by the U.S. Government is subject to restrictions as set forth in FAR §52.227-14 Alternates I, II, and III (JUN 1987); FAR §52.227-19 (JUN 1987) and/or FAR §12.211/12.212 (Commercial Technical Data/Computer Software); and DFARS §252.227-7015 (NOV 1995) (Technical Data) and/or DFARS §227.7202 (Computer Software), as applicable.  Contractor/Manufacturer is ESRI, 380 New York Street, Redlands, CA 92373-8100, USA.

ESRI, ARC/INFO, ArcCAD, ArcIMS, ArcView, BusinessMAP, MapObjects, PC ARC/INFO, SDE, and the ESRI globe logo are trademarks of ESRI, registered in the United States and certain other countries; registration is pending in the European Community. 3D Analyst, ADF, the ARC/INFO logo, AML, ArcNews, ArcTIN, the ArcTIN logo, ArcCOGO, the ArcCOGO logo, ArcGrid, the ArcGrid logo, ArcInfo, the ArcInfo logo, ArcInfo Librarian,  ArcInfo—Professional GIS, ArcInfo—The World's GIS, ArcAtlas, the ArcAtlas logo, the ArcCAD logo, the ArcCAD WorkBench logo, ArcCatalog, the ArcData logo, the ArcData Online logo, ArcDoc, ArcEdit, the ArcEdit logo, ArcEditor, ArcEurope, the ArcEurope logo, ArcExplorer, the ArcExplorer logo, ArcExpress, the ArcExpress logo, ArcFM, the ArcFM logo, the ArcFM Viewer logo, ArcGIS, the ArcGIS logo, the ArcIMS logo, ArcNetwork, the ArcNetwork logo, ArcLogistics, the ArcLogistics Route logo, ArcMap, ArcObjects, ArcPad, the ArcPad logo, ArcPlot, the ArcPlot logo, ArcPress, the ArcPress logo, the ArcPress for ArcView logo, ArcReader, ArcScan, the ArcScan logo, ArcScene, the ArcScene logo, ArcSchool, ArcSDE, the ArcSDE logo, the ArcSDE CAD Client logo, ArcSdl, ArcStorm, the ArcStorm logo, ArcSurvey, ArcToolbox, ArcTools, the ArcTools logo, ArcUSA, the ArcUSA logo, ArcUser, the ArcView logo, the ArcView GIS logo, the ArcView 3D Analyst logo, the ArcView Business Analyst logo, the ArcView Data Publisher logo, the ArcView Image Analysis logo, the ArcView Internet Map Server logo, the ArcView Network Analyst logo, the ArcView Spatial Analyst logo, the ArcView StreetMap logo, the ArcView StreetMap 2000 logo, the ArcView Tracking Analyst logo, ArcVoyager, ArcWorld, the ArcWorld logo, Atlas GIS, the Atlas GIS logo, AtlasWare, Avenue, the Avenue logo, the BusinessMAP logo, the Data Automation Kit logo, Database Integrator, DBI Kit, the Digital Chart of the World logo, the ESRI Data logo, the ESRI Press logo, ESRI—Team GIS, ESRI—The GIS People, FormEdit, Geographic Design System, Geography Matters, GIS by ESRI, GIS Day, GIS for Everyone, GISData Server, InsiteMAP, MapBeans, MapCafé, the MapCafé logo, the MapObjects logo, the MapObjects Internet Map Server logo, ModelBuilder, MOLE, the MOLE logo, NetEngine, the NetEngine logo, the PC ARC/INFO logo, PC ARCEDIT, PC ARCPLOT, PC ARCSHELL, PC DATA CONVERSION, PC NETWORK, PC OVERLAY, PC STARTER KIT, PC TABLES, the Production Line Tool Set logo, RouteMAP, the RouteMAP logo, the RouteMAP IMS logo, Spatial Database Engine, the SDE logo, SML, StreetEditor, StreetMap, TABLES, The World's Leading Desktop GIS, Water Writes, and Your Personal Geographic Information System are trademarks; and ArcData, ArcOpen, ArcQuest, ArcWatch, ArcWeb, Rent-a-Tech, Geography Network, the Geography Network logo, www.geographynetwork.com, www.gisday.com, @esri.com, and www.esri.com are service marks of ESRI.

Other companies and products mentioned herein are trademarks or registered trademarks of their respective trademark owners.

Table of Contents

Welcome to Arc Hydro (version 1.0 Beta 2)	5

Arc Hydro Data Model	6

About Arc Hydro Data Model	6

Network	8

About Network Components	8

HydroNetwork	9

HydroEdge	9

HydroJunction	11

SchematicLink	12

SchematicNode	13

HydroEvent	14

HydroPointEvent	15

HydroLineEvent	16

Drainage	17

About Drainage Components	17

DrainagePoint	18

DrainageLine	19

Catchment	20

Watershed	22

Basin	23

Channel	25

About Channel Components	25

ProfileLine	26

CrossSection	27

CrossSectionPoint	28

Hydrography	29

About Hydrography Components	29

HydroResponseUnit	30

Dam	30

Bridge	31

Structure	32

MonitoringPoint	33

WaterWithdrawal	34

WaterDischarge	35

UserPoint	36

Waterbody	37

HydroLine	38

Time Series	40

About Time Series Components	40

Arc  Hydro Tools	42

About Arc Hydro Tools	42

Tools Overview	44

Concepts	44

Database Design	45

Tags	50

Terrain Preprocessing	53

About Terrain Preprocessing	53

Data Management  - Terrain Preprocessing	54

DEM Reconditioning (AGREE)	56

Fill Sinks	57

Flow Direction	58

Flow Accumulation	59

Stream Definition	60

Stream Segmentation	60

Catchment Grid Delineation	61

Catchment Polygon Processing	61

Drainage Line Processing	62

Adjoint Catchment Processing	63

Drainage Point Processing	64

Watershed Processing	66

About Watershed Processing	66

Data Management - Watershed Processing	66

Batch Watershed Delineation	68

Batch Subwatershed Delineation	70

Drainage Area Centroid	70

Longest Flow Path	71

Attribute Tools	73

About Attribute Tools	73

Data Management - Attribute Tools	74

Assign HydroID	75

Generate From/To Node for Lines	76

Find Next Downstream Line	77

Calculate Length Downstream for Edges	78

Calculate Length Downstream for Junctions	79

Find Next Downstream Junction	81

Store Area Outlets	82

Consolidate Attributes	88

Accumulate Attributes	88

Network Tools	89

About Network Tools	89

Data Management - Network Tools	89

Hydro Network Generation	90

Node/Link Schema Generation	93

Store Flow Direction	94

Set Flow Direction	95

ApUtilities	97

About ApUtilities	97

List Maps	98

Add New Map	98

Set Target Locations	99

XML Manager	99

Load/Merge Config XML	100

HydroID Tables Manager	100

Arc Hydro Help	101

Buttons and Tools	102

About Buttons and Tools	102

Flow Path Tracing	103

Point Delineation	104

Batch Point Generation	105

Assign Related Identifier	106

Global Point Delineation	107

Trace By NextDownID Attribute	109

# Welcome to Arc Hydro (version 1.0 Beta 2)

Arc Hydro is an ArcGIS-based system geared to support water resources applications.  It consists of two key components:

Arc Hydro Data Model

Arc Hydro Tools

The Arc Hydro data model provides a basic database design for water resources.  The Arc Hydro Tools are a series of tools built on top of the Arc Hydro database that facilitates the analyses often performed in the water resources area.

The data model and the tools provide a foundation for water resources applications in GIS that can be built upon to meet the desired requirements.  The Arc Hydro model and tools are designed to be flexible and easy to customize.

# Arc Hydro Data Model

## About Arc Hydro Data Model

The ArcGIS Hydro data model, often shortened to Arc Hydro, describes geospatial and temporal data on surface water resource features of the landscape.  The data model addresses three issues:

- Hydro Description – What are the principal water resource features of the landscape?

- Hydro Connectivity – How does water move from feature to feature?

- Hydro Modeling – What are the time patterns of water flow and water quality associated with these features?

The ArcGIS Hydro data model describes only natural water systems, and does not support constructed water infrastructure, for which there is a separate data model, ArcFM Water.  Moreover, the model does not currently support features describing aquatic geology, geomorphology, and groundwater systems.

An Arc Hydro geodatabase consists of Hydro Features connected to Time Series.  Hydro Features describe the physical environment through which water flows, and the Time Series describe the flow and water quality properties of the water within those features.  Every Hydro Feature is uniquely defined within an Arc Hydro geodatabase by a HydroID, and associations are formed between features by storing the HydroID of one feature as an attribute of another.  These linkages can be used to trace water movement from one feature to the next, and also to associate several different geospatial representation of the hydrologic entity with one another.  Time series are connected to Hydro Features by storing the HydroID of the feature as an attribute of each time series data value.

The Arc Hydro data model consists of five components: Network, Drainage, Channels, Hydrography and Time Series:

- The Network component contains a water resources network of streams, rivers and the centerlines of water bodies.  Its main purpose is to describe the connectivity of water movement through the landscape.

- The Drainage component defines drainage areas delineated through analysis of land surface topography.

- The Channel component describes the three-dimensional shape of river and stream channels.

- The Hydrography component contains base map information on point, line and area water resource features.

- The Time Series component describes time varying water property of the features.

Details on the data model are published in the Water Resources (Hydro) Data Model, available at http://arconline.esri.com/arconline/datamodels/water.cfm or through the Water Resources Consortium at http://www.crwr.utexas.edu/giswr/*!Inet("http://www.crwr.utexas.edu/giswr/.

## Network

### About Network Components

The Network feature dataset describes the connectivity of water flow through the landscape as a water resources network.

HydroNetwork is the principal feature class of this dataset: it is an ArcGIS geometric network, whose components are HydroEdges and HydroJunctions.  Water flows along HydroEdges, and HydroEdges are connected by HydroJunctions.  The Hydro Network describes flow through rivers and streams, and the centerlines of waterbodies.

SchematicNetwork, which is composed of the SchematicLink and SchematicNode feature classes are used to symbolize the connection of drainage areas to HydroJunctions, and to provide a simplified view of water flow through the landscape.

HydroEvents stores tabular information on points or lines associated by linear referencing with the Hydro Network.

Network Components

- HydroNetwork

- HydroEdge

- HydroJunction

- SchematicLink

- SchematicNode

- HydroEvent

- HydroPointEvent

- HydroLineEvent

### HydroNetwork

HydroNetwork is a geometric network composed of the HydroEdge and HydroJunction feature classes from the Network dataset.  The network can be created by using the Case Tool, in ArcCatalog or with the Arc Hydro Tools function Hydro Network Generation.

A geometric network stores the connectivity between its features.

### HydroEdge

The HydroEdge feature class contains lines connected by junctions in the HydroNetwork.    The HydroEdge class is built as a ComplexEdgeFeature so that HydroJunctions can be added to the interior of HydroEdges without necessarily splitting the HydroEdges.  HydroEdges are PolylineM features, which means that their vertices have (x,y,m) coordinates, where m is the measure location along the HydroEdge.

HydroEdges are subtyped into flowlines and shorelines.  Water can flow only along Flowlines and not along Shorelines.

- Flowlines:  single-line streams, centerlines of double-line streams, and centerlines of waterbodies)

- Shorelines (i.e., banks of double-line streams, shorelines of inland waterbodies, or coastlines of the sea or ocean).

The HydroEdge PolylineM Complex Edge feature class defines the following attributes:

- HydroID – integer: A unique feature identifier within a geodatabase.

- HydroCode – string: A permanent, public identifier of the Hydro Feature, or a unique identifier of a Hydro Feature among a set of Arc Hydro geodatabases.

- ReachCode – string: Reach code.  Corresponds to the DrainID in the DrainageLine feature class, referring to the DrainID of the catchment where the reach is located.

- Name – string: The geographic name of the feature.

- LengthKm – double: The length of the HydroEdge given in kilometers independent of the map units.  This is to ensure that when length is used as weight in network solvers, a realistic value is available, regardless of the map projection of the data.

- LengthDown – double: The distance from the downstream end of a HydroEdge along the shortest path to the nearest network sink (Calculate Length Downstream for Edges).

- FlowDir – integer: This attribute is defined by the HydroFlowDirections Coded Value Domain.  It indicates the direction of flow along the HydroEdge.  The Coded Values are: Uninitialized = 0, WithDigitized = 1, AgainstDigitized = 2, and Indeterminate = 3, where WithDigitized means that the flow direction is in the same direction as the direction in which the HydroEdge was digitized, and AgainstDigitized means the converse, where flow direction is opposite to the direction of digitizing of the HydroEdge.  The initial value given is 1.

- FType – string:  Describes the feature type of the HydroEdge.  This attribute can be expanded using a Coded Value Domain to represent feature types such as Natural Channel, Constructed Channel, Pipeline, Connector, Bankline, Shoreline and Coastline.  Defaults to Synthetic Channel.

- EdgeType – Subtype Coded Value Domain (HydroEdgeType): Determines if the HydroEdge is a Flowline or a Shoreline with an initial value of 1.  Coded Values are: Flowline = 1 and Shoreline = 2.

- Enabled – Coded Value Domain: This attribute is inherited from the Network Feature parent class of HydroEdge.  Coded Values are: Disabled = 0 and Enabled = 1.  The initial value is 1.  The Enabled attribute is used to disable portions of the stream hydrography that appears in maps but are not needed to trace water movement through the landscape.  It provides an alternative to deleting these features from the Hydro Network.  Examples are minor loops on streams, and isolated small channel reaches that are unconnected with the rest of the Hydro Network.  When a network feature is disabled, it acts as a barrier when the network is traced and no flow can pass through it.

Subtypes: Flowline and Shoreline

Relationships: None

### HydroJunction

The HydroJunction class includes junctions between HydroEdges and other points that are vital to a network analysis, such as outlet points for drainage areas and locations of stream gages or other point features.  HydroJunctions are points that stand for sources, sinks, stream junctions and other relevant user-defined locations.  Among the HydroJunctions, sinks play a significant role in the analysis because each feature in the network drains to a single Sink.

The HydroJunction Simple Junction feature class defines the following attributes:

- HydroID – integer: A unique identifier within a geodatabase.

- HydroCode – string: A permanent, public identifier of the Hydro Feature, or a unique identifier of a Hydro Feature among a set of Arc Hydro geodatabases.

- NextDownID – integer: Identifies the HydroID of the next downstream HydroJunction.

- LengthDown – double: The distance downstream from the HydroJunction along the shortest path to the nearest network sink (Calculate Length Downstream for Junctions).

- DrainArea – double: The total drainage area upstream of the HydroJunction.

- FType – string: Describes the feature type of the HydroJunction.  This attribute can be expanded using a Coded Value Domain to describe various junction feature types such as Stream Confluence, Drainage Outlet, Monitoring Point, Dam, etc. (Null for beginning of streams for the entire drainage area considered).

- Enabled – Coded Value Domain: An Attribute inherited from the Network Feature parent class of HydroJunction that indicates if the junction participates in the network.  Coded Values are: Disabled = 0 and Enabled = 1.  Initial value = 1.  If a junction is disabled, no flow may pass through it.  A skeleton Hydro Network of just the main rivers and streams can be identified by selecting a threshold DrainArea, and disabling all junctions having a DrainArea smaller than the threshold value.

- AncillaryRole – Coded Value Domain: An Attribute inherited from the Junction Feature parent class of HydroJunction that describes the role of the junction in the network.  Coded Values are: None = 0, Source = 1, and Sink = 2.  Initial value is 0.  When a geometric network is constructed, those HydroJunctions at the downstream outlets of the network must selected as sinks and have their Ancillary Role set to Sink or 2.  The flow direction on HydroEdges is  then assigned using the network analyst tools, and additional editing, if necessary, so as to allow flow towards these sinks.

Subtypes: None

Relationships: WaterbodyHasJunction is a 1 to 1 relationship between HydroJunction (using HydroID) and Waterbody (using JunctionID); and WatershedhasJunction is a 1 to 1 relationship between HydroJunction (using HydroID) and Watershed (using JunctionID).

### SchematicLink

The SchematicLink and SchematicNode classes together make up a Schematic Network.  A schematic network can be drawn to connect objects among several feature classes of the Arc Hydro geodatabase to provide a simplified but more general type of connectivity among water features than is provided by the Hydro Network.  The Schematic Network is made up of simple point and line feature classes, but may be built into a geometric network if the user desires.  The Schematic Network is used to symbolize the connection of drainage areas to the HydroJunctions, and to create a simplified view of water movement through the landscape by means of straight-line connections between selected junctions in the Hydro Network.  The Schematic Network uses standard FromNode-ToNode topology to describe its connectivity.

The SchematicLink Polyline feature class defines the following attributes:

- HydroID – integer: A unique feature identifier within a geodatabase.

- HydroCode – string: A permanent, public identifier of the Hydro Feature, or a unique identifier of a Hydro Feature among a set of Arc Hydro geodatabases.

- FromNodeID – integer: Indicates the HydroID  of the SchematicNode at the upstream end of the Link.

- ToNodeID – integer: Indicates the HydroID of the Schematic Node at the downstream end of the Link.

Subtypes: None

Relationships: Node

### SchematicNode

The SchematicNode Point feature class contains the points in a Schematic Network, which may represent any feature within an Arc Hydro geodatabase.  Typical types of SchematicNodes are: drainage area centroids, drainage area outlets, and stream confluences.

The SchematicNode Point feature class defines the following attributes:

- HydroID – integer: A unique feature identifier within a geodatabase.

- HydroCode – string: A permanent, public identifier of the Hydro Feature, or a unique identifier of a Hydro Feature among a set of Arc Hydro geodatabases.

- FeatureID – integer: HydroID of the HydroFeature from which the node was created.  This enables the SchematicNode to connect to the attribute information of the feature it describes.  If necessary, a relationship can be built using these attributes.

Subtypes: None

Relationships: None

### HydroEvent

The HydroEvent abstract object class stores attributes and methods for events.  In some network applications, it is desirable to specify the location of a point along the river not as a pair of Cartesian coordinates, but as an address on the network, which is called linear reference.  This address is given by the combination of a ReachCode and a measure value on that reach, such as the percent distance from the bottom end of the reach at which the point is located.  This is analogous to specifying that a house has a street address of 123 Oak Ave., rather than giving its latitude and longitude.  Events are located using linear referencing and display tabular information that is of interest to the user.  A typical example is a Wastewater discharge event, whose attributes might include the volume of the discharge and its water quality.  As a default, it is assumed that HydroEvents are defined on the lines contained in the HydroEdge feature class.  However, ProfileLine and CrossSection also possess the PolylineM geometry type necessary for linear referencing of events, so HydroEvents can be created on those feature classes also.

The HydroEvent abstract object class defines the following attributes:

- ReachCode – string: The attribute that identifies the line on which the measure is located.  This attribute can be changed to RiverCode if the measure is in river miles or kilometers and events are defined on ProfileLines rather than HydroEdges.

Subtypes: None

Relationships: None

### HydroPointEvent

HydroPointEvent is a point event, which means a set of attributes attached to a single location on a line.  HydroPointEvents can represent many things as the user can imagine in relation to natural water systems.  Some examples are wastewater discharge points, USGS gage stations, and irrigation diversions.

The HydroPointEvent object class defines the following attribute:

- ReachCode – string: The attribute that identifies the line on which the measure is located.  This attribute can be changed to RiverCode if the measure is in river miles or kilometers and events are defined on ProfileLines rather than HydroEdges.

- Measure – double: The measure of the location of the event as determined by linear referencing.

Subtypes: None

Relationships: None

### HydroLineEvent

The HydroLineEvent, similar to HydroPointEvent, inherits from the HydroEvent object class and has a ReachCode attribute to locate the event on a specific HydroEdge.  The Fmeasure and Tmeasure attributes give the precise location of the event on the HydroEdge.  When a linear event spans a group of reaches, a separate HydroLineEvent object is needed for each reach.  ArcGIS does not explicitly support the concept of a grouped event, and if the user wants to create such a group, an additional attribute is needed on the HydroLineEvent indicating to which group the event belongs.

The HydroLineEvent object class has the following attributes:

- ReachCode – string: The attribute that identifies the line on which the measure is located.  This attribute can be changed to RiverCode if the measure is in river miles or kilometers and events are defined on ProfileLines rather than HydroEdges.

- FMeasure – double: Measure location of the start of the line event.

- TMeasure – double: Measure location of the end of the line event.

- Offset – double: Offset distance of the event from the line defining its measures.  This is used to allow display of multiple line events on the same line without having them overlapping the map display.  An example might be where an economic analysis of flood damage requires a damage reach defined on the left bank of the channel, and another on the right bank.

Subtypes: None

Relationships: None

## Drainage

### About Drainage Components

The Drainage system of the landscape defines the direction of surface water flow according to land surface topography.  Drainage divides defined by ridge lines separate the area draining to one stream from the adjacent areas draining to neighboring streams.

Drainage Area is a generic term used within the Arc Hydro data model to describe any feature class describing such drainage areas, and it is sub classed into Catchment, Watershed and Basin, to describe specific types of drainage areas.  Accurate drainage boundaries are essential for hydrologic modeling studies.  Drainage boundaries may be delineated manually from a topographic map, digitized from digital raster graphic map (DRG), or determined through the use of raster data from Digital Elevations Models.

Drainage Lines are lines defining the drainage network of the landscape.  Drainage Lines are closely related to but not necessarily coincident with the HydroEdges in the Hydro Network.  Indeed, Drainage Lines can be built into an excellent Hydro Network if a suitable mapped stream network is unavailable.  Drainage Lines may be defined during the terrain analysis using Digital Elevation Models by using the Arc Hydro Tools function Drainage Line Processing.

Drainage Points are outlet points for the Drainage Areas on the Drainage Lines.

The following Drainage Components are described in more details:

- Drainage Point

- Drainage Line

- Catchment

- Watershed

- Basin

### DrainagePoint

DrainagePoint is a subclass of DrainageFeature.  A DrainagePoint represents the point at the center of a Digital Elevation Model cell at the most downstream location within a Drainage Area.  The Drainage Area is associated with its Drainage Point using the DrainID attribute that both class inherit from the Drainage Feature class.  Drainage Points are also known as Seed Points, Outlet Points, or Pour Points of Drainage Area.

The DrainagePoint Point feature class defines the following attributes:

- HydroID – integer: A unique identifier within a geodatabase.

- HydroCode – string: A permanent, public identifier of the Hydro Feature, or a unique identifier of a Hydro Feature among a set of Arc Hydro geodatabases.

- DrainID – integer: Provides a link between point, line and area features of a drainage system, and allows the vector features in the Arc Hydro data model to be linked numerically to the raster cells used to create them in Digital Elevation Model analysis.  Examples include GridCode for DEM processing in the ArcGIS spatial analyst, and integer equivalents of the Pfaffstetter Code, Hydrologic Unit Code, and ReachCode, which are standard hydrologic descriptors of Catchments, Watersheds and Reaches, respectively.

- JunctionID – integer: The HydroID of the HydroJunction associated with this DrainagePoint.  This is the HydroJunction, which serves as the outlet of this Drainage Area onto the Hydro Network.

Subtypes: None

Relationships: None

### DrainageLine

The DrainageLine is the line through the centers of the Digital Elevation Model (DEM) cells on a drainage path.  It is produced when the DEM-based drainage paths are vectorized.  DrainagePoints lie on DrainageLines.  If necessary, the DrainageLines and DrainagePoints can be built into a geometric network and used as the Hydro Network for an Arc Hydro geodatabase.

The DrainageLine Polyline feature class defines the following attributes.  The user can add attributes as necessary.

- HydroID – integer: A unique identifier within a geodatabase.

- HydroCode – string: A permanent, public identifier of the Hydro Feature, or a unique identifier of a Hydro Feature among a set of Arc Hydro geodatabases.

- DrainID – integer: Provides a link between point, line and area features of a drainage system, and allows the vector features in the Arc Hydro data model to be linked numerically to the raster cells used to create them in Digital Elevation Model analysis.  Examples include GridCode for DEM processing in the ArcGIS spatial analyst, and integer equivalents of the Pfaffstetter Code, Hydrologic Unit Code, and ReachCode, which are standard hydrologic descriptors of Catchments, Watersheds and Reaches, respectively.

Subtypes: None

Relationships: None

### Catchment

Catchment is a polygon subclass of DrainageArea.  A Catchment feature class is an elementary drainage area produced by subdivision of the landscape using a consistent set of physical rules.  Typically, this subdivision is carried out by defining a stream network from a threshold flow accumulation on a Digital Elevation Model, and then delineating a Catchment for each stream segment in the network.

The Catchment Polygon feature class defines the following attributes.  The user can add attributes as necessary.

- HydroID – integer: A unique identifier within a geodatabase.

- HydroCode – string: A permanent, public identifier of the Hydro Feature, or a unique identifier of a Hydro Feature among a set of Arc Hydro geodatabases.

- DrainID – integer: Provides a link between point, line and area features of a drainage system, and allows the vector features in the Arc Hydro data model to be linked numerically to the raster cells used to create them in Digital Elevation Model analysis.  Examples include GridCode for DEM processing in the ArcGIS spatial analyst, and integer equivalents of the Pfaffstetter Code, Hydrologic Unit Code, and ReachCode, which are standard hydrologic descriptors of Catchments, Watersheds and Reaches, respectively.

- AreaSqKm – double: Drainage Area in square kilometers.  This is so that the earth surface area is known for the Drainage Area regardless of the map projection of the data.

- JunctionID – integer: The HydroID of the associated HydroJunction, which is the outlet on the Hydro Network where flow from this Drainage Area discharges to the stream or river system.

- NextDownID – integer: The HydroID of the next downstream area in this drainage area class.  This is used to support Area to Area navigation within a set of drainage areas.

Subtypes: None

Relationships: None

### Watershed

The Watershed feature class is a subclass of Drainage Area, which contains a landscape subdivision into human-selected drainage areas, which may drain to a point on a river network, to a river segment or to a waterbody.  By contrast to Catchments, which can be automatically delineated using a set of rules applied to a terrain model, the definition of Watersheds requires a human intervention process, where the analyst selects and edits the Watershed subdivision of the landscape unit so that the desired arrangement is obtained.

The Watershed Polygon feature class defines the following attributes.  The user can add attributes as necessary.

- HydroID – integer: A unique identifier within a geodatabase.

- HydroCode – string: A permanent, public identifier of the Hydro Feature, or a unique identifier of a Hydro Feature among a set of Arc Hydro geodatabases.

- DrainID – integer: Provides a link between point, line and area features of a drainage system, and allows the vector features in the Arc Hydro data model to be linked numerically to the raster cells used to create them in Digital Elevation Model analysis.  Examples include GridCode for DEM processing in the ArcGIS spatial analyst, and integer equivalents of the Pfaffstetter Code, Hydrologic Unit Code, and ReachCode, which are standard hydrologic descriptors of Catchments, Watersheds and Reaches, respectively.

- AreaSqKm – double: Drainage Area in square kilometers.  This is so that the earth surface area is known for the Drainage Area regardless of the map projection of the data.

- JunctionID – integer: The HydroID of the associated HydroJunction, which is the outlet on the Hydro Network where flow from this Drainage Area discharges to the stream or river system.

- NextDownID – integer: The HydroID of the next downstream area in this drainage area class.  This is used to support Area to Area navigation within a set of drainage areas.

Subtypes: None

Relationships: WatershedhasJunction is a 1 to 1 relationship between HydroJunction (using HydroID) and Watershed (using JunctionID).

### Basin

Basin is an administratively chosen standardized watershed used for reference and data cataloging, usually named after the principal rivers and streams of a region.  Basins may define the geographic extent of an Arc Hydro dataset, and thus constitute standardized watershed template for data archiving and delivery.  Basin is a polygon subclass of DrainageArea.  Basins usually contain sets of Watersheds and Catchments.

The Basin Polygon feature class defines the following attributes.  The user can add attributes as necessary.

- HydroID – integer: A unique identifier within a geodatabase.

- HydroCode – string: A permanent, public identifier of the Hydro Feature, or a unique identifier of a Hydro Feature among a set of Arc Hydro geodatabases.

- DrainID – integer: Provides a link between point, line and area features of a drainage system, and allows the vector features in the Arc Hydro data model to be linked numerically to the raster cells used to create them in Digital Elevation Model analysis.  Examples include GridCode for DEM processing in the ArcGIS spatial analyst, and integer equivalents of the Pfaffstetter Code, Hydrologic Unit Code, and ReachCode, which are standard hydrologic descriptors of Catchments, Watersheds and Reaches, respectively.

- AreaSqKm – double: Drainage Area in square kilometers.  This is so that the earth surface area is known for the Drainage Area regardless of the map projection of the data.

- JunctionID – integer: The HydroID of the associated HydroJunction, which is the outlet on the Hydro Network where flow from this Drainage Area discharges to the stream or river system.

- NextDownID – integer: The HydroID of the next downstream area in this drainage area class.  This is used to support Area to Area navigation within a set of drainage areas.

Subtypes: None

Relationships: None

## Channel

### About Channel Components

The Channel feature dataset provides a three-dimensional representation of the river and stream channel shape, which is used for studies of flood inundation, stream ecology and morphology.  Two feature classes are defined under Channel feature ProfileLine and CrossSection, and one object class, CrossSectionPoint.  ProfileLines are lines drawn parallel to the stream flow, such as the stream thalweg line and banklines.  CrossSections are drawn transverse to the streamflow.  Both feature classes are derived from the ChannelFeature abstract class, which is itself derived from the HydroFeature class.  Channel information can be collected in the field using surveying techniques, or by extracting the data from digital terrain models (DTMs) in the form of a triangulated irregular network (TIN), or digital elevation model (DEM).

### ProfileLine

A ProfileLine is a longitudinal view of a channel, using lines drawn parallel to the stream flow.  Various types of ProfileLines can be drawn:

- The Thalweg is the ProfileLine through the lowest point of the stream channel at each CrossSection.

- Banklines show the intersection of the water surface and the land surface on each side of the channel.

- Floodlines can be drawn in the flood plain to symbolize the main direction of water flow when the flood plain is inundated.

- Streamlines can be drawn anywhere in a water flow, for example to depict the water surface profile for a particular design discharge in the channel.

Typically, the representative ProfileLine is the channel thalweg.  Thalwegs and bank lines can be digitized from high resolution digital orthographic photos or maps, or manually surveyed.  As with the CrossSection feature class, the ProfileLine feature class is a 3-D PolylineM class, which means that its vertices have (x,y,z,m) coordinates.  In this case, the m-coordinate is the ProfileM or measure value such as river mile or kilometer, or stationing or chainage in feet or meters from some upstream or downstream reference point on the channel.

The ProfileLine 3-D PolylineM feature class defines the following attributes:

- FType – string: A descriptor specifying the type of feature represented by a ProfileLine.  Possible values are Thalweg, bankline, floodline or streamline.  If necessary, these values can be specified as coded value domain, and used to subtype the ProfileLine features.

- ProfOrigin – string: Describes the origin of the data used to form the ProfileLine.

Subtypes: None

Relationships: None

### CrossSection

A CrossSection in ArcGIS is a 3-D PolylineM feature, where each vertex in the line is defined by four coordinates: x, y, z, m. The (x,y) coordinates give the location of the vertex in the horizontal plane, z represents the elevation above a vertical datum, and m is the CrossSection measure (CrossM), or distance along the CrossSection in the (x,y) plane.

The CrossSection 3-D PolylineM feature class defines the following attributes:

- CSCode – string: This is a user defined cross section identifier that is unique for each cross section, usually formed as the concatenation of the river, reach, and ProfileLine measure value e.g. "Waller Creek, Middle Fork, 132.3"

- JunctionID – integer: the HydroID of the HydroJunction at the equivalent hydrologic location to this CrossSection.  This attribute is provided so that the CrossSections can be related to the corresponding location on the Hydro Network for purposes of linking hydrologic and hydraulic modeling of water flows

### CrossSectionPoint

The CrossSectionPoint object class stores cross-sectional point data, and by means of CSCode, can be associated with a CrossSection feature.  While a single cross-sectional line represents the location of a cross section, several cross section points (a one-to-many relationship exists between CrossSection feature line and CrossSectionPoints) allow for a geometric description of the channel.  If the cross section geometry is stored as CrossSectionPoints, the elevation (z) and measure (m) field values of a CrossSection feature line should be set to NaN (not a number) to avoid confusion.

The CrossSectionPoint object class defines the following attributes:

- CSCode – string: A user-defined cross section identifier that is unique for each cross section.

- CrossM – double: The measure, or m value, in feet or meters or percent distance, of a Cross Section Point along a cross section for which elevation values are known.

- Elevation – double: Stores the elevation, or value, of a CrossSectionPoint above a vertical datum.

Subtypes: None

Relationships: CrossSectionHasPoints is a one-to-many relationship between CrossSection (using CSCode) and CrossSectionPoint (using CSCode).

## Hydrography

### About Hydrography Components

The Hydrography feature dataset contains the map representation of the surface water features.  It contains several kinds of simple point, line and area features:

- HydroPoint, HydroLine and HydroArea features which are derived from the "blue lines" or hydrography layer of topography maps

- Waterbody features, such as lakes, bays and estuaries

- Point features derived from tabular data inventories, such as dams, bridges, structures, monitoring points (gages and sampling points), points of water withdrawal and water discharge, and user points for any other purpose.  The latitude and longitude attributes of these tabular data inventories are used to define the location of the point features.

- HydroResponseUnit features which describe the hydrologic character of the land surface from the viewpoint of the partitioning of surface water balance accounting.

### HydroResponseUnit

HydroResponseUnit describes the hydrologic character of the land surface for surface water balance accounting.  Typically, HydroResponseUnits are formed by the intersection of soil and land cover polygons, but these units may also be defined by climate cells, administrative or drainage basin boundaries, and aquifer boundaries.  Each HydroResponseUnit is considered to have uniform and representative properties to describe the partitioning at the land surface of precipitation into runoff, evaporation and infiltration.

The HydroResponseUnit Polygon feature class defines the following attribute:

- AreaSqKm – double: The area calculated in square kilometers independent of map units.  This attribute is included so that regardless of the map projection, the earth surface area is known for each unit to permit  correct computation of the surface water balance.

Subtypes: None

Relationships: None

### Dam

A dam is a structure that creates an artificial lake, or reservoir, by blocking a river or stream.  Dams may harness the energy of falling water or provide flood control.  They also store water for municipal water supply and crop irrigation, raise the water level to allow for navigation, and divert water into a pipe or channel.  Dams are such important structures that it is common for them to be described by a tabular data inventory containing the latitude and longitude, from which the point feature can be created.

The Dam Point feature class does not define any additional attributes.  Dams are complex objects that typically have many descriptive attributes.  The user can add these as necessary.

Subtypes: None

Relationships: None

### Bridge

A bridge is a structure that allows passage over an obstacle.  Bridges carry railroad lines, highways, and pathways over water and deep gorges.  Bridges impede water flow by narrowing the stream cross-sectional area, which increases the water surface elevation and produces backwater effects upstream of the bridge.  Information on bridge features is also maintained as part of the transportation network, and a simple definition of bridge locations is the set of points created by the intersection of the transportation and stream networks.  A culvert is a road conveyance over a stream formed by a set of pipes inserted into the road embankment.  A bridge differs from a culvert in that it is a formal structure crossing the stream, and connected to road embankments at both ends of the structure.  In the Arc Hydro data model, culverts are considered a subset of bridges, and the FType attribute inherited from the Hydrography abstract feature class can be used to distinguish bridges and culverts, or different types of bridges, if necessary.

The Bridge Point feature class does not define any additional attributes.

Subtypes: None

Relationships: None

### Structure

The Structure class contains any other kind of water resource structure that is not represented by Dam or Bridge classes.  Structures change the hydraulic properties of the flow though the network by their presence.  Typical examples of structures include detention ponds on small streams, levees designed to hold back floodwaters, and weirs.  These can be also natural features like waterfalls if they have significant effect on the hydraulic properties of the network.  The Structure class can also be used to describe buildings and other physical structures in the flood plain for purposes of economic analysis of the effects of flood damage.

The Structure Point feature class does not define any additional attributes.

Subtypes: None

Relationships: None

### MonitoringPoint

Monitoring points are intended to store the locations of gages that measure water quantity or quality, and may have time series associated with them for analysis purposes.  Monitoring points may also be subtypes or subclassed by the user.  Examples of monitoring points include water quality monitoring stations, stream gage stations, rain gage stations, and any other type of fixed-location data collection points.

The MonitoringPoint feature class does not define any additional attributes, although typically there are many such attributes, including the name of the agency maintaining the monitoring site.  The user can add such attributes as necessary.

Subtypes: None

Relationships: None

### WaterWithdrawal

The WaterWithdrawal feature class represents points at which flow is diverted or pumped from surface water systems, or pumped from aquifers through groundwater wells.  The point locations and associated flow data are maintained by government agencies that issue water rights, or the legal authority for individuals or institutions to withdraw water from natural water systems.  These points are significant because they represent an interface between the human use of water and the natural water systems, and they are important in computing the water balance.

The WaterWithdrawal Point feature class does not define any additional attributes.

Subtypes: None

Relationships: None

### WaterDischarge

Water discharge points add flow to the stream network.  Data on water discharge points is maintained by government agencies issuing permits for such discharges.  Water can be discharged from wastewater treatment plants, by return flow from irrigation systems, or other sources.  Typically, water discharges degrade the quality of the receiving waters, so these points are important for computing the water balance and water quality of natural water systems.

The WaterDischarge Point feature class does not define any additional attributes.

Subtypes: None

Relationships: None

### UserPoint

The UserPoint class contains any points of interest not described by other point hydrography feature classes.  User points may include locations where rivers cross aquifer, political or administrative boundaries, or define major confluence points on the river network.  The UserPoint class is a good place to load large data sets of various kinds of points so that after application of the Arc Hydro schema, the points can be organized and exported to other classes as appropriate.

The UserPoint feature class does not define any additional attributes.

Subtypes: None

Relationships: None

### Waterbody

The Waterbody class is subclass of HydroArea, which represents water bodies such as lakes, bays and estuaries.  The distinction between the Waterbody and HydroArea feature classes is that a waterbody may be a very complex spatial feature with many islands, and even components water bodies.  The Waterbody feature class provides a generalized representation of waterbodies, for which additional detail is contained in the HydroArea class, if necessary.

The Waterbody Polygon feature class defines the following attributes:

- AreaSqKm – double: The waterbody area calculated in square kilometers independent of map units.

- JunctionID – integer: HydroID of the HydroJunction, which is the outlet of this waterbody.

Subtypes: None

Relationships: WaterbodyhasJunction is a 1 to 1 relationship between HydroJunction (using HydroID) and Waterbody (using JunctionID).

### HydroLine

The HydroLine class is designed to contain line features that are important for the cartographic representation of the area of study and are no contained in the Hydro Network.  Some examples of hydrographic lines are natural streams and rivers, manmade canals or ditches, pipelines that carry water underground, connectors that are used when the original data had some obstruction covering the hydrologic feature, and artificial paths which represent the centerlines of lakes and other water bodies.  Isolated ponds and lakes which are not part of the river network, shorelines, islands boundaries, no wake zones, swimming and recreation areas, roads, county and state boundary lines, jurisdictional boundaries for river authorities, and city limits are all marked off by lines which are important for cartography.  They serve to provide a spatial reference for viewers of the data and so are necessary in the model.  These types of lines are stored in the Hydrography subclass HydroLine.  A subset of the above features may be built into the Hydro Network, in which case these features should be deleted from the HydroLine feature class to avoid duplicate representation of the same features.

The HydroLine Polyline feature class does not define any additional attributes.

Subtypes: None

Relationships: None

## Time Series

### About Time Series Components

The geospatial feature of the Arc Hydro data model describe the water environment, that is, the physical environment through which water flows.  Also important are the water properties at any geographic location: its discharge, water surface elevation, and water quality.  These properties are contained in the TimeSeries component of the data model.  At present, this component consists of a single object representing time series in a very generalized way.  Any number of time series data of any type describing any Arc Hydro feature can be stored in this object.  ArcGIS version 8.1 does not have functions specifically designed to support manipulation of time series so full implementation of time series requires custom coding of a time series toolset operating on top of the Arc Hydro data model.  A time series connected to a spatial feature can be thought of as a time-varying attribute value of that feature.  Within the geodatabase, time series data are treated like any other tabular data.  It should be noted that this TimeSeries object exists to connect temporal and geospatial water resources data in a single geodatabase.  It is not intended to be a fully configured water resources time series database, which requires a more complicated structure than the one described here.

The TimeSeries object class defines four attributes:

- FeatureID – integer: The HydroID of the HydroFeature described by a time series.

- TSType – string: The type of time series data (such as precipitation, streamflow or evaporation).

- TSDateTime – datetime: The date and time of the beginning of the time interval for which the TSValue applies. The format is YYYY-MM-DD hh:mm:ss.sss TZ where:

- YYYY: 4-digit year

- MM: month (01 to 12)

- DD: day (01 to 31)

- hh: hour (00 to 23)

- mm: minutes (00 to 59)

- ss.sss: seconds with milliseconds (00.000 to 59.999)

- TZ: time zone, which can be local time zone, or ISO format (+0800, -0500, etc.)

- TSValue – double: actual time series data value.

Subtypes: None

Relationships: None.  A relationship can be built between the feature and the time series associated with that feature if necessary.

# Arc  Hydro Tools

## About Arc Hydro Tools

The Arc Hydro tools are a set of public domain utilities developed on top of the Arc Hydro data model.  They operate in the ArcGIS environment.  Some of the functions require the Spatial and 3D Analyst extensions.

The tools are accessed through the Arc Hydro Tools toolbar, where they are grouped by functions into five menus, and six buttons.

- Terrain Preprocessing: functions dealing with Digital Elevation Model (DEM) processing.  They are mostly used once in order to prepare spatial information for later use.

- Watershed Processing: functions dealing with watershed and subwatershed delineation, and basin characteristic determination.  They operate on top of the spatial data prepared in the terrain preprocessing stage.

- Attribute Tools: functions allowing to generate key attributes (fields) in the Arc Hydro data model.  Some of the tools require the existence of a geometric network.

- Network Tools:  functions allowing to generate or manipulate properties of geometric (hydro) network.

- ApUtilities: functions allowing to manage Arc Hydro project properties.  In general, they will be seldom used.

- Buttons and Tools

- Flow Path Tracing

- Point Delineation

- Batch Point Generation

- Assign Related Identifier

- Global Point Delineation

- Trace By NextDownID Attribute

The tools have two key purposes.  The first purpose is to manipulate (assign) key attributes in the Arc Hydro data model.  These attributes form the basis for further analyses.  They include the key identifiers (such as HydroID, DrainID, NextDownID, etc.) and the measure attributes (such as LengthDown).  The second purpose for the tools is to provide some core functionality often used in water resources applications.  This includes DEM-based watershed delineation, network generation, and attribute-based tracing.

The functionality of Arc Hydro tools is expected to grow over time.  They have been implemented in a way that allows easy addition to their functionality, either internally (by adding additional code) or externally, by providing additional functionality using key Arc Hydro data structures.

## Tools Overview

### Concepts

The Arc Hydro tools are based on the following concepts:

- Map

- XML Document

- Tag

- Target Location

Map: A map is a synonym for data frame.  Arc Hydro operates in the active map that is shown in bold in the ArcMap Table of Contents: only layers belonging to this map may be used as inputs.

Limitation: Arc Hydro does not support a map/data frame, whose name contains space(s) unless it was created using the function ApUtilities>Add New Map.

Tag: Arc Hydro uses tags to pass the inputs, i.e. the layers associated to the tags, to the functions.  Tags are assigned to the selected layers when the functions run.  They may also be assigned, reassigned, or reset (set to Null) with the Data ManagementData_Management functions.

Tags assignment are specific to a data frame/map: the Data Management functions manage the layers/tags for the active data frame only.  Tags make it possible to start from any function as long as the input layers meet all the requirements for the function.  These requirements are detailed in the help topics associated to each function.

XML document: a default XML document is associated with the Arc Hydro Tools – this document is called ArcHydroTools.xml.  When a new map document containing the Arc Hydro tools is created, a XML is automatically associated with this new map document.  It contains default configuration parameters, and will be updated as the tools are used to reflect the user’s configuration.

This file is not saved explicitly on disk, but is part of the map document.  This XML may be displayed and edited using the XML Manager.

Target Location: the outputs to the functions are created in default target locations, which may be modified using the function ApUtilities>Set Target Locations.  Information on the default locations is available in the help topic associated to that function.

### Database Design

The following themes are used by Arc Hydro.

#### GRIDs

#### DEM Grid

- Value: Elevation of the cell of the DEM from a defined datum.

- Count: Number of cells in the Grid with that value item.

AgreeDEM Grid (DEM Reconditioning)

- Value: Elevation of the cell of the DEM processed by the AGREE method.

- Count: Number of cells in the Grid with that value item.

HydroDEM Grid (Fill Sinks)

- Value: Elevation of the cell from a filled DEM.

- Count: Number of cells in the Grid with that value item.

FlowDirGrid (Flow Direction)

- Value: Direction of the steepest descent from that cell according to the D8 method.

- Count: Number of cells in the Grid with that value item.

FlowAccGrid (Flow Accumulation)

- Value: Number of cells flowing into that particular cell.

- Count: Number of cells in the Grid with that value item.

Stream Grid (Stream Definition)

- Value: ‘1’ for the cells in the input Flow Accumulation Grid that have a value greater than the threshold used to define the streams.  All other cells in the stream Grid contain no data.

- Count: Number of cells in the Grid belonging to the identified streams.

LinkGrid (Stream Segmentation)

- Value: The value in each cell in a segment between the segment junctions or in head segments represents a unique Grid code, and all the cells in that particular segment have that same Grid code.

- Count: Number of cells in the Grid with that value item.

CatchmentGrid (Catchment Grid Delineation)

- Value: The value in each catchment is the value of the stream link in the Link Grid into which the catchment drains.

- Count: Number of cells in the Grid with that value item.

#### Vector Feature Classes

Catchment (Catchment Polygon Processing)

- GRIDCODE – Double: Alias of GridCode generated by the application during the conversion from Catchment Grid into Catchment polygon. GRIDCODE is the ‘Value’ of the corresponding Catchment Grid.

- Shape_Length – Double: Perimeter in map units.

- Shape_Area – Double: Area in map units.

- GridID – Long: GridCode of the corresponding Catchment Grid.

- HydroID – Long: Unique identifier in the Hydro database.  System generated.

- DrainID – Long: Unique identifier  (HydroID) of the drainage area linked to the feature.  In this particular case, Catchment being a drainage area, same as HydroID (field may be created empty by Adjoint Catchment Processing).

- NextDownID – Long: HydroID of the next downstream catchment (field may be created empty by Drainage Line Processing if the Catchment tag is set, created/populated by Adjoint Catchment Processing or Hydro Network Generation).

- JunctionID – Long: HydroID of the junction that serves as the outlet for the catchment (field may be created empty by Drainage Line Processing ,if the Catchment tag is set, or by Adjoint Catchment Processing , created/populated by Hydro Network Generation or Store Area Outlets).

DrainageLine (Drainage Line Processing)

- ARCID – Long: Unique identifier of the drainage line segment in the feature class.

- GRID_CODE – Long: Grid code of the catchment the drainage line segment belongs to.

- FROM_NODE – Long: From Node for the drainage line segment.

- TO_NODE – Long: To Node for the drainage line segment.

- Shape_Length – Double: Length of the river segment in map units.

- GridID – Long: Grid code of the catchment the drainage line segment belongs to.

- HydroID – Long: Unique identifier in the Hydro database.

- NextDownID – Long: HydroID of the next downstream drainage line.  Populated with "-1" if there is no next downstream line.

- DrainID – Long: HydroID of the catchment where the drainage line is located. Populated with "-9" if there is no associated catchment (created by Adjoint Catchment Processing).

AdjointCatchment (Adjoint Catchment Processing)

- Shape_Length – Double: Perimeter in map units.

- Shape_Area – Double: Area in map units.

- GridID – Long: GridID of the catchment located downstream from the Adjoint catchment.

DrainagePoint (Drainage Point Processing)

- HydroID – Long: Unique identifier in the Hydro database. System generated.

- DrainID – Long: HydroID of the catchment draining to this drainage point. Populated with "-9" if there is no associated catchment (created empty, created/populated by Hydro Network Generation).

- GridID – Long: GridID of the catchment draining to the point.

- JunctionID – Long: HydroID of the Hydro Junction corresponding to the Drainage Point.  One Hydro Junction may correspond to several Drainage Points (created by Hydro Network Generation).

BatchPoint (Batch Point Generation)

- Name – String (25): Name of the batch point.

- Description – String (25): Description of the batch point.

- BatchDone – Short: Indicates whether the Batch Watershed Delineation was performed:

o	‘0’ Delineation not performed

o	‘1’ Delineation already performed

o	‘-1’ Delineation failed

- SnapOn – Short: Indicates whether the point is to be snapped during the Batch Watershed Delineation.

o	‘0’: do not snap

o	‘1’: snap

- HydroID – Long: Unique identifier of the Batch Point in the Arc Hydro database.

Watershed (Batch Watershed Delineation)

- DrainID – Long: Same as HydroID, since watershed is a drainage area.

- Name – String (25): Name retrieved from the Batch Point input layer.

- Comment – String (25): Comments retrieved from the Batch Point input layer.

- HydroID – Long: Unique identifier of the watershed in the Hydro database.

WatershedPoint (Batch Watershed Delineation)

- DrainID – Long: HydroID of the associated watershed.

- Name – String (25): Name retrieved from the Batch Point input layer.

- HydroID – Long: Unique identifier of the watershed point in the Hydro database.

SubWatershed (Batch Subwatershed Delineation)

- GRIDCODE – Long: ID of the Batch Point used to generate the watershed.

- HydroID – Long: Unique identifier of the subwatershed in the Hydro database.

- DrainID – Long: Same as HydroID since subwatershed is a drainage area.

Centroid (Drainage Area Centroid)

- DrainID – Long: HydroID of the drainage area associated with the centroid.

LongestFlowPath (Longest Flow Path)

- DrainID – Long: HydroID of the drainage area associated with the centroid.

HydroEdge (Hydro Network Generation)

- Shape_Length – Double: Length of the edge feature in map units.

- HydroID – Long: Unique identifier of the HydroEdge in the Hydro database.

- ReachCode – Long: Reach code.  Correspond to the DrainID in the DrainageLine feature class, referring to the DrainID of the catchment where the reach is located.

- FlowDir – Short: Flow direction for the edge. Defaults to WithDigitized flow direction.

- FType – String (25): Feature Type.  Defaults to Synthetic Channel.

- EdgeType – Short: Type of Edge. Available values are Flowline and Shoreline.  Default to Flowline.

- Enabled – Short: Indicates whether the feature is enabled (True) or disabled (False) in the geometric network.  A feature that is disabled in the network acts as a barrier when the network is traced.

- LengthDown – Double: Length from the edge to the downstream sink the edge flows to (Calculate Length Downstream for Edges).

HydroJunction (Hydro Network Generation)

- HydroID – Long: Unique identifier of the HydroJunction in the Hydro database.

- NextDownID – Long: HydroID of the next downstream HydroJunction.

- FType – String (25): Feature type of the HydroJunction: Stream Confluence, Drainage Outlet, Null for beginning of streams (for the entire drainage area considered).

- Enabled – Short: Indicate whether the feature is enabled (True) or disabled (False) in the geometric network.  A feature that is disabled in the network acts as a barrier when the network is traced.

- AncillaryRole – Short: Specify whether the feature represents a source, sink or neither.  Used to calculate the flow direction in the geometric network (1: Source, 2: Sink (Drainage Outlet), 0: None).

- LengthDown – Double: Length from the junction to the downstream sink the junction flows to (Calculate Length Downstream for Junctions).

SchemaLink (Node/Link Schema Generation)

- HydroID – Long: Unique identifier of the Schema Link in the Hydro database.

- FromNodeID – Long: Hydroid of the Schema Node at the upstream end of the Schema Link.

- ToNodeID – Long: HydroID of the Schema Node at the downstream end of the Schema Link.

- LinkType – Short: 1 for Watershed link, 2 for Junction link.

SchemaNode (Node/Link Schema Generation) *WAM_Schema_Generation

- HydroID – Long: Unique identifier of the Schema Node in the Hydro database.

- FeatureID – Long: HydroID of the HydroFeature from which the node was created.  This enables the schematic node to connect to the attribute information of the feature it describes.  If necessary, a relationship can be built using these attributes

- SrcType – Short: Source Type: 1 for Watershed, 2 for Junction.

GlobalWatershed (Global Point Delineation)

- DrainID – Long: Unique identifier  (HydroID) of the drainage area linked to the feature.  In this particular case, GlobalWatershed being a drainage area, same as HydroID.

- Name – String (25): Name entered by the user during the delineation.

- Comment – String (25): Comments about the global watershed.

- HydroID – Long: Unique identifier of the GlobalWatershed in the Hydro database.

GlobalWatershedPoint (Global Point Delineation)

- DrainID – Long: Unique identifier (HydroID) of the drainage area (GlobalWatershed) linked to the feature.

- Name – String (25): Name entered by the user during the delineation.

- HydroID – Long: Unique identifier of the GlobalWatershedPoint in the Hydro database.

### Tags

Tags are "names" associated with themes that identify the role of the theme in the application.  When Arc Hydro is looking for a data source to be used for a particular function it looks for a tag and not a theme name.  Once the tag is found, the theme associated with that tag is identified and passed on to the function. This allows flexibility in theme naming as the theme names can be fully user defined (but the tags are not) and changed at any time without affecting the application performance.

A layer may have several roles (and hence associated tags), but a tag can have one and only one associated layer.  The following table describes the tags/roles used by the Arc Hydro tools.

| Tag | Type | Output from | Input to |
| --- | --- | --- | --- |
| Agree Catchment | Polygon | Adjoint Catchment Processing | Batch Watershed Delineation Point Delineation |
| Agree DEM | Grid | DEM Reconditioning |  |
| Agree Stream | Line |  | DEM Reconditioning |
| Area | Polygon |  | Store Area Outlets |
| Batch Point | Point | Batch Point Generation | Batch Watershed Delineation |
| Catalog Unit Edge | Edge |  | Global Point Delineation |
| Catalog Unit Junction | Junction |  | Global Point Delineation |
| Catalog Unit Polygon | Polygon |  | Global Point Delineation |
| Catchment | Polygon | Catchment Polygon Processing | Adjoint Catchment Processing Batch Watershed Delineation Hydro Network Generation Point Delineation |
| Catchment Grid | Grid | Catchment Grid Delineation | Catchment Polygon Processing Drainage Point Processing |
| Centroid | Point | Drainage Area Centroid |  |
| DEM | Grid |  | Fill Sinks |
| Drainage Area |  |  | Drainage Area Centroid Longest Flow Path |
| Drainage Line | Line | Drainage Line Processing | Adjoint Catchment Processing Hydro Network Generation |
| Drainage Point | Point | Drainage Point Processing | Store Area Outlets Hydro Network Generation |
| Flow Accumulation Grid | Grid | Flow Accumulation | Stream Definition Drainage Point Processing |
| Flow Direction Grid | Grid | Flow Direction | Flow Accumulation Stream Segmentation Catchment Grid Delineation Drainage Line Processing Batch Watershed Delineation Batch Subwatershed Delineation Longest Flow Path Flow Path Tracing Point Delineation |
| Global Watershed Point | Point | Global Point Delineation |  |
| Global Watershed Point | Point | Global Point Delineation |  |
| Hydro DEM | Grid | Fill Sinks | Flow Direction |
| Hydro Edge | Edge | Hydro Network Generation | Calculate Length Downstream for Edges Store Flow Direction Set Flow Direction |
| Hydro Junction | Junction | Hydro Network Generation | Calculate Edge Downstream for Junctions Find Next Downstream Junction Store Area Outlets |
| Junctions | Junction |  | Node/Link Schema Generation |
| Line | Line |  | Generate From/To Node for Line Find Next Downstream Line |
| Link Grid | Grid | Stream Segmentation | Catchment Grid Delineation Drainage Line Processing |
| Longest Flow Path | Line | Longest Flow Path |  |
| Raw DEM | Grid |  | DEM Reconditioning |
| Schema Link | Line | Node/Link Schema Generation |  |
| Schema Node | Point | Node/Link Schema Generation |  |
| Stream Grid | Grid | Stream Definition | Stream Segmentation Batch Watershed Delineation Point Delineation |
| Subwatershed | Polygon | Batch Subwatershed Delineation |  |
| Subwatershed Outlet | Point |  | Batch Subwatershed Delineation |
| Watershed | Polygon | Batch Watershed Delineation Point Delineation |  |
| Watershed Point | Point | Batch Watershed Delineation Point Delineation |  |
| Watershed Polygons | Polygon |  | Node/Link Schema Generation |

## Terrain Preprocessing

### About Terrain Preprocessing

The purpose of terrain preprocessing is to perform an initial analysis of the terrain and to prepare the dataset for further processing.  A Digital Elevation Model (DEM) of the study area is required as input for terrain preprocessing: a DEM is a grid in which each cell is assigned the average elevation on the area represented by the cell.  The DEM must be in ESRI GRID format.

During the processing, potential problems with the terrain representation can be identified, thus preventing the DEM errors from propagating to the later stages of the analysis.  A successful preprocessing is an indication that the underlying DEM does not contain major problems that will prevent further analyses.

The initial basin delineation that is performed during the preprocessing has no meaning for later basin processing (except for performance during the extraction stage), since all parameters can be changed.  In general, the recommended size for stream threshold definition (which in turn defines the sub basin delineation during preprocessing) is 1% of the overall area.  For increased performance on large DEMs (over 20,000,000 cells), the size of the threshold may be reduced.

Note: Arc Hydro does not provide specific tools for DEM editing and modification.  Standard Spatial Analyst functionality can be used for such purposes.

#### Terrain Preprocessing contains the following functions

- Data Management: set the tags for the themes used in the Terrain Preprocessing menu.

- DEM Reconditioning (AGREE): enforce linear drainage pattern (vector) onto a DEM (grid).  Implement AGREE methodology.

- Fill Sinks: fill sinks for an entire DEM (grid).

- Flow Direction: create flow direction grid from a DEM grid.

- Flow Accumulation: create flow accumulation grid from a flow direction grid.

- Stream Definition: create a new grid (stream grid) with cells from a flow accumulation grid that exceed user-defined threshold.

- 	Stream_definition

- Stream Segmentation: create a stream link grid from the stream grid (every link between two stream junction gets a unique identifier).

- Catchment Grid Delineation: create a catchment grid for segments in the stream link grid.  It identifies areas draining into each stream link.

- Catchment Polygon Processing: create catchment polygons out of the catchment grid.

- Drainage Line Processing: create streamlines out of the stream link grid.

- Adjoint Catchment Processing: create adjoint catchment polygon for each catchment in the catchment polygon feature class.  Adjoint catchment is total upstream area (if any) draining into a single catchment.

- Drainage Point Processing: create a drainage point at the most downstream point in the catchment (center of a grid cell with the largest value in the flow accumulation grid for that catchment).

### Data Management  - Terrain Preprocessing

Arc Hydro manages the input/output to the tools by using tags that are automatically assigned by the functions to the selected inputs and outputs.  A tag may be used as input by one function and as output by another one.  For example, the "Flow Direction Grid" tag is an output from Flow Direction, and an input to Flow Accumulation.

The Data Management function in the Terrain Preprocessing menu provides a global view of the tags assignments for that menu in the active Map/Data Frame.  The function also allows assigning, reassigning or resetting the tags.  A tag may be reset by selecting "Null" as the corresponding layer.  When a reset tag is used as output, the function presents the user with the default layer name associated to the tag.  This default name is defined in the XML file and may be modified (see XML Manager).

#### How the function works

- The pull down menu next to each tag offers for selection only those themes in the active data frame that match the required theme type (line, point, polygon, grid).

- A "Null" option is available in the list.  Selecting "Null" remove the theme assigned to that role.

- A layer may be assigned several tags.  When a layer is used with an additional tag, the user is warned that at least one other tag has already been assigned to this theme.

- tag may be assigned to one and only layer at any time.

#### Tags defined in Terrain Preprocessing

| Tag | Output from | Input to |
| --- | --- | --- |
| Raw DEM |  | DEM Reconditioning |
| Agree Stream |  | DEM Reconditioning |
| Agree DEM | DEM Reconditioning |  |
| DEM |  | Fill Sinks |
| Hydro DEM | Fill Sinks | Flow Direction |
| Flow Direction Grid | Flow Direction | Flow Accumulation |
|  |  | Stream Segmentation |
|  |  | Catchment Grid Delineation |
|  |  | Drainage Line Processing |
| Flow Accumulation Grid | Flow Accumulation | Stream Definition |
|  |  | Drainage Point Processing |
| Stream Grid | Stream Definition | Stream Segmentation |
| Link Grid | Stream Segmentation | Catchment Grid Delineation |
|  |  | Drainage Line Processing |
| Catchment Grid | Catchment Grid Delineation | Catchment Polygon Processing |
|  |  | Drainage Point Processing |
| Catchment | Catchment Polygon Processing | Adjoint Catchment Processing |
| Drainage Line | Drainage Line Processing | Adjoint Catchment Processing |
| Adjoint Catchment | Adjoint Catchment Processing |  |
| Drainage Point | Drainage Point Processing |  |

### DEM Reconditioning (AGREE)

The DEM Reconditioning function (Terrain Preprocessing menu) modifies Digital Elevation Models (DEMs) by imposing linear features onto them (burning/fencing).  This function is an implementation of the AGREE method developed by Ferdi Hellweger at the University of Texas at Austin in 1997.  For a full reference to the procedure refer to the web link http://www.ce.utexas.edu/prof/maidment/GISHYDRO/ferdi/research/agree/agree.html.

The function needs two inputs that both have to be present in the map document:

| Input |  | Output |
| --- | --- | --- |
| "Raw DEM" Grid |  | "Agree DEM" Grid |
| "Agree Stream" Grid |  |  |

#### Input/Output Management

If there is a grid theme that has a tag "Raw DEM", it will be used as a default for the input grid.  If not, the user needs to select an existing grid theme name that will be tagged with the "Raw DEM" tag at the end of the operation.

If there is a linear feature class that has a tag "Agree Stream", it will be used as a default for the input feature class.  If not, the user needs to select an existing linear feature class that will be tagged with the "Agree Stream" tag at the end of the operation.

If there is a grid theme that has a tag "Agree DEM", it will be used as a default for the output grid.  If not, the user needs to specify a grid name that will be tagged with the "Agree DEM" tag at the end of the operation.  If the specified output grid already exists, the user is prompted whether to remove the existing dataset.

After initiating the function, the user needs to enter three reconditioning parameters:

- Vector buffer (cells) – this is the number of cells around the linear feature class for which the smoothing will occur.

- Smooth drop/raise – this is the amount (in vertical units) that the river will be dropped (if the number is positive) or the fence extruded (if the number is negative).  This value will be used to interpolate DEM into the buffered area (between the boundary of the buffer and the dropped/raised vector feature).

- Sharp drop/raise – this is the additional amount (in vertical units) that the river will be dropped (if the number is positive) or the fence extruded (if the number is negative).  This has the effect of additional burning/fencing on top of the smooth buffer interpolation.  It needs to be performed to ensure preserving the linear features used for burning/fencing.

The values used for the AGREE parameters depend on the nature of the DEM and the issues that are being resolved.  In many cases, a trial and error approach is needed before satisfactory results are obtained.  Refer to the original paper for some guidelines on the initial parameter selection.

The reconditioned AGREE DEM should be processed with the Fill Sinks function to ensure that the potential sinks generated in the streams are removed.  The resulting filled DEM will be used to compute the flow direction/flow accumulation.

Note: Topographic analyses should be conducted on the original "Raw DEM"  (before reconditioning), to ensure that correct elevations are extracted (e.g. for slope computation).

### Fill Sinks

The Fill Sinks function (Terrain Preprocessing menu) fills sinks in a grid.  If a cell is surrounded by higher elevation cells, the water is trapped in that cell and cannot flow.  The Fill Sinks function modifies the elevation value to eliminate these problems.

The function takes as input a DEM grid ("DEM" tag), which can be either an unprocessed DEM or a reconditioned DEM created with the DEM Reconditioning function ("AgreeDEM " tag).  The function produces as output a grid ("Hydro DEM" tag) where no sinks exist.

| Input |  | Output |
| --- | --- | --- |
| "DEM" Grid |  | "Hydro DEM" Grid |

#### Input/Output Management

If there is a grid theme that has a tag "DEM", it will be used as a default for the input grid.  If not, the user needs to specify an existing grid theme name that will be tagged with the "DEM" tag at the end of the operation.

If there is a grid theme that has a tag "Hydro DEM", it will be used as a default for the output grid.  If not, the user needs to specify a grid name that will be tagged with the "Hydro DEM" tag at the end of the operation.  If the specified output grid already exists, the user is prompted whether to remove the existing dataset.

Filling sinks is an iterative process that can be time-consuming.  The status of the processing will be displayed in the ArcMap status bar.

Filling sinks is an operation that needs to be executed with care.  The resulting DEM will have no inner depressions, that is, all the runoff from the DEM will reach its edges.  In most cases, this is a correct assumption (when depressions are the artifacts of DEM generation), but in some cases that is not correct (e.g. inner lakes).  In such cases, the hydrologically correct DEM needs to be developed in a different manner outside of Arc Hydro, and provided to Arc Hydro for processing.  The Fill Sinks operation should not be performed, but rather the hydrologically correct DEM should be used in place of the "Hydro DEM".

### Flow Direction

The Flow Direction function (Terrain Preprocessing menu) takes a grid ("Hydro DEM" tag) as input, and computes the corresponding flow direction grid ("Flow Direction Grid" tag).  The values in the cells of the flow direction grid indicate the direction of the steepest descent from that cell.

| Input |  | Output |
| --- | --- | --- |
| "Hydro DEM" Grid |  | "Flow Direction Grid" |

#### Input/Output Management

If there is a grid theme that has a tag "Hydro DEM", it will be used as a default for the input grid.  If not, the user needs to select an existing grid theme name that will be tagged with the "Hydro DEM" tag at the end of the operation.

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the output grid.  If not, the user needs to specify a grid name that will be tagged with the "Flow Direction Grid" tag at the end of the operation.  If the specified output grid already exists, the user is prompted whether to remove the existing dataset.

### Flow Accumulation

The Flow Accumulation function (Terrain Preprocessing menu) takes as input a flow direction grid ("Flow Direction Grid" tag).  It computes the associated flow accumulation grid ("Flow Accumulation Grid" tag) that contains the accumulated number of cells upstream of a cell, for each cell in the input grid.

| Input |  | Output |
| --- | --- | --- |
| "Flow Direction Grid" |  | "Flow Accumulation Grid" |

#### Input/Output Management

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the input grid.  If not, the user needs to select an existing grid theme name that will be tagged with the "Flow Direction Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Flow Accumulation Grid", it will be used as a default for the output grid.  If not, the user needs to specify a grid name that will be tagged with the "Flow Accumulation Grid" tag at the end of the operation.  If the specified output grid already exists, the user is prompted whether to remove the existing dataset.

Flow accumulation processing is the most time consuming task in the terrain preprocessing and can take a lot of time to complete.  It requires significant computer memory (at least 64MB of RAM, preferably more) and a significant amount of hard disk space (about 5 times the size of the final flow accumulation GRID).  If the function fails to operate properly, the most likely reason is the lack of hard disk space.  The lack of memory can greatly increase the time required for processing.

### Stream Definition

The Stream Definition function (Terrain Preprocessing menu) takes a flow accumulation grid ("Flow Accumulation Grid" tag) as input and creates a Stream Grid ("Stream Grid" tag) for a user-defined threshold.  The stream grid contains a value of "1" for all the cells in the input grid that have a value greater than the given threshold.  All other cells in the Stream Grid contain no data.

| Input |  | Output |
| --- | --- | --- |
| "Flow Accumulation Grid" |  | "Stream Grid" |

#### Input/Output Management

If there is a grid theme that has a tag "Flow Accumulation Grid", it will be used as a default for the input grid.  If not, the user needs to select an existing grid theme name that will be tagged with the "Flow Accumulation Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Stream Grid", it will be used as a default for the output grid.  If not, the user needs to specify a grid name that will be tagged with the "Stream Grid" tag at the end of the operation.  If the specified output grid already exists, the user is prompted whether to remove the existing dataset.

The user will then be prompted to enter a threshold to define the stream.  This threshold is defined as a number of cells: the default is 1% of the maximum flow accumulation value.

### Stream Segmentation

The Stream Segmentation function (Terrain Preprocessing menu) creates a grid of stream segments that have a unique identification.  Either a segment may be a head segment, or it may be defined as a segment between two segment junctions.   All the cells in a particular segment have the same grid code that is specific to that segment.

| #### Input |  | #### Output |
| --- | --- | --- |
| "Flow Direction Grid" Grid |  | "Link Grid" Grid |
| "Stream Grid" Grid |  |  |

#### Input/Output Management

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the first input grid.  If not, the user needs to select an existing grid theme (of flow direction type) that will be tagged with the "Flow Direction Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Stream Grid", it will be used as a default for the second input grid.  If not, the user needs to select an existing grid theme that will be tagged with the "Stream Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Link Grid", it will be used as a default for the output grid.  If not, the user needs to specify a grid name that will be tagged with the "Link Grid" tag at the end of the operation.  If the output grid already exists, the user is prompted whether to remove the existing dataset.

### Catchment Grid Delineation

The Catchment Grid Delineation function (Terrain Preprocessing menu) creates a grid in which each cell carries a value (grid code) indicating to which catchment the cell belongs.  The value corresponds to the value carried by the stream segment that drains that area, defined in the stream segment link grid.

| Input |  | Output |
| --- | --- | --- |
| "Flow Direction Grid" |  | "Catchment Grid" |
| "Link Grid" |  |  |

#### Input/Output Management

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the first input grid.  If not, the user needs to select an existing grid theme (of flow direction type) that will be tagged with the "Flow Direction Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Link Grid", it will be used as a default for the second input grid.  If not, the user needs to specify a grid name that will be tagged with the "Link Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Catchment Grid", it will be used as a default for the output grid.  If not, the user needs to specify a grid name that will be tagged with the "Catchment Grid" tag at the end of the operation.  If the output grid already exists, the user is prompted whether to remove the existing dataset.

### Catchment Polygon Processing

The Catchment Polygon Processing function (Terrain Preprocessing menu) takes as input a catchment grid (‘Catchment Grid" tag) and converts it into a catchment polygon feature class ("Catchment" tag).  The adjacent cells in the grid that have the same grid code are combined into a single area, whose boundary is vectorized.  The single cell polygons and the "orphan" polygons generated as the artifacts of the vectorization process are dissolved automatically, so that at the end of the process there is just one polygon per catchment.

| Input |  | Output |
| --- | --- | --- |
| "Catchment Grid" |  | "Catchment" Polygon Feature Class |

Input/Output Management

If there is a grid theme that has a tag "Catchment Grid", it will be used as a default for the input grid.  If not, the user needs to select an existing grid theme name that will be tagged with the "Catchment Grid" tag at the end of the operation.

If there is a polygon feature class that has a tag "Catchment", it will be used as a default for the output feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Catchment" tag at the end of the operation.  If the output feature class already exists, the user is prompted whether to remove the existing dataset.

Fields created:

- 	GRIDCODE: Alias of GridCode, generated by the application during the conversion from Catchment grid to Catchment polygon.  GRIDCODE is the ‘Value’ of the corresponding Catchment Grid.

- 	GridID: GridCode of the corresponding Catchment grid.

- 	HydroID: Unique identifier in the Hydro database. System generated.

- 	DrainID: Unique identifier (HydroID) of the drainage area linked to the feature.  Same as HydroID since Catchment is a drainage area.

### Drainage Line Processing

The Drainage Line Processing function (Terrain Preprocessing menu) converts the input Stream Link grid into a Drainage Line feature class.  Each line in the feature class carries the identifier of the catchment in which it resides.

| Input |  | Output |
| --- | --- | --- |
| "Link Grid" |  | "Drainage Line" Feature Class |
| "Flow Direction Grid" |  |  |

#### Input/Output Management

If there is a grid theme that has a tag "Link Grid", it will be used as a default for the first input grid.  If not, the user needs to select a grid name that will be tagged with the "Link Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the second input grid.  If not, the user needs to select an existing grid theme (of flow direction type) that will be tagged with the "Flow Direction Grid" tag at the end of the operation.

If there is a theme that has a tag "Drainage Line", it will be used as a default for the output line feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Drainage Line" tag at the end of the operation.  If the output feature class already exists, the user is prompted whether to remove the existing dataset.

Fields created:

- ARCID – Unique identifier of the drainage line in the feature class.

- GRID_CODE – Grid code of the catchment the drainage line segment belongs to.

- FROM_NODE – From Node number of the drainage line segment.

- TO_NODE – To Node number of the drainage line segment.

- GridID – Grid code of the catchment the drainage line segment belongs to.

- HydroID – Unique identifier of the drainage line in the Hydro database.

- NextDownID – HydroID of the next downstream drainage line.  Populated with "-1" if there is no next downstream line.

#### Note

One of the tasks performed by this function is the identification of upstream-downstream relationship.  In rare cases, this relationship cannot be determined automatically based on connectivity and DEM, and the user will be asked to identify whether a segment is an outlet or not.  This situation usually occurs when a drainage line segment is very short and the elevation at its beginning and end is the same, thus preventing the application from identifying the correct directionality.

In such cases, the questionable segment will be highlighted, the application will zoom on it, and an input box will be brought up.  The input box enables the user to zoom in or out of the segment and specify whether the segment is an outlet (most of the times) or not.  This process is repeated for every such segment until the directionality is fully established.

#### The function may also create the two following empty fields in the feature class tagged as "Catchment":

- NextDownID – HydroID of the next downstream catchment.

- JunctionID – HydroID of the junction that serves as the outlet for the catchment.

### Adjoint Catchment Processing

The Adjoint Catchment Processing function (Terrain Preprocessing menu) generates the aggregated upstream catchments from the "Catchment" feature class.  For each catchment that is not a head catchment, a polygon representing the whole upstream area draining to its inlet point is constructed and stored in a feature class that has an "Adjoint Catchment" tag.  This feature class is used to speed up the point delineation process.

| Input |  | Output |
| --- | --- | --- |
| "Drainage Line" Feature Class |  | "Adjoint Catchment" Feature Class |
| "Catchment" Feature Class |  |  |

#### Input/Output Management

If there is a linear feature class that has a tag "Drainage Line", it will be used as a default for the first input theme.  If not, the user needs to select an existing linear feature class that will be tagged with the "Drainage Line" tag at the end of the operation.

Required fields: HydroID, DrainID, GridID, From_Node, To_Node, NextDownID

- Field created: DrainID – HydroID of the Catchment in which the drainage line is located.

If there is a polygon feature class that has a tag "Catchment", it will be used as a default for the second input theme.  If not, the user needs to select an existing polygon feature class that will be tagged with the "Catchment" tag at the end of the operation.

Required fields: HydroID, GridID

Fields created:

- DrainID – created empty (if not previously existing)

- JunctionID – created empty (if not previously existing)

- NextDownID – HydroID of the next down catchment.  Populated with "-1" if there is no dowstream catchment.

If there is a polygon feature class that has a tag "Adjoint Catchment", it will be used as a default for the output feature class.  If not, the user needs to specify a feature class name that will be tagged with the "AdjointCatchment" tag at the end of the operation.  If the output feature class already exists, the user is prompted whether to remove the existing dataset.

Field created: GridID – GridID of the catchment located downstream from the Adjoint Catchment.

### Drainage Point Processing

The Drainage Point Processing function (Terrain Preprocessing menu) allows generating the drainage points associated to the catchments.

| Input |  | Output |
| --- | --- | --- |
| "Flow Accumulation Grid" |  | "Drainage Point" Feature Class |
| "Catchment Grid" |  |  |

Input/Output Management

If there is a grid theme that has a tag "Flow Accumulation Grid", it will be used as a default for the first input grid.  If not, the user needs to select an existing grid theme (of type flow accumulation) that will be tagged with the "Flow Accumulation Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Catchment Grid", it will be used as a default for the second input grid.  If not, the user needs to select a grid name that will be tagged with the "Catchment Grid" tag at the end of the operation.

If there is a point feature class that has a tag "Drainage Point", it will be used as a default for the output point feature class.  If not, the user needs to specify a point feature class name that will be tagged with the "Drainage Point" tag at the end of the operation.  If the output feature class already exists, the user is prompted whether to remove the existing dataset.

Fields created:

- HydroID: Unique identifier of the drainage point in the Hydro database. System generated.

- DrainID: HydroID of the catchment draining to this drainage point (this field is created empty, and may be populated by Hydro Network Generation).

- GridID: GridID of the catchment draining to the drainage point.

## Watershed Processing

### About Watershed Processing

The Watershed Processing menu provides access to several functions that allow fast watershed delineation and topographic characteristics extraction.  The following functions are available.

- Data Management: set the tags for the themes used in the Watershed Processing menu.

- Batch Watershed Delineation: create a watershed for every point in the batch point feature class.  Results are stored in a watershed (polygon) feature class.  Watersheds are overlapping (if points are on the same stream).

- Batch Subwatershed Delineation: create a subwatershed for every point in the batch point feature class.  Results are stored in a subwatershed (polygon) feature class.  Subwatersheds are non-overlapping (if points are on the same stream).Batch_Subwatershed_Delineation

- Drainage Area Centroid: create a point at the centroid of each polygon in a polygon feature class and store it in a point feature class.

- Longest Flow Path: create a line following the longest flow path based on the steepest descent (as defined by the flow direction grid) in a catchment of watershed.

### Data Management - Watershed Processing

Arc Hydro manages the input/output to the tools by using tags that are automatically assigned by the functions to the selected inputs and outputs.  A tag may be used as input by one function and as output by another one.  For example, the "Flow Direction Grid" tag is an input from Flow Direction, and an input to Batch Watershed Delineation.

The Data Management function in the About_Basin_PreprocessingWatershed Processing menu provides a global view of the tags assignments for that menu in the active Map/Data Frame.  The function also allows assigning, reassigning or resetting the tags.  A tag may be reset by selecting "Null" as the corresponding layer.  When a reset tag is used as output, the function presents the user with default layer name associated to the tag.  This default is defined in the XML file and may be modified (see XML Manager).

#### How the function works

- The pull down menu next to each tag offers for selection only those themes in the active data frame that match the required theme type (line, point, polygon, grid).

- A "Null" option is available in the list.  Selecting "Null" remove the theme assigned to that role.

- A layer may be assigned several tags.  When a layer is used with an additional tag, the user is warned that at least one other tag has already been assigned to this theme.

- tag may be assigned to one and only layer at any time.

#### Tags defined in Watershed Processing

| Tag | Output from | Input to |
| --- | --- | --- |
| Flow Direction Grid |  | Batch Watershed Delineation |
|  |  | Batch Subwatershed Delineation |
|  |  | Longest Flow Path |
| Stream Grid |  | Batch Watershed Delineation |
| Catchment |  | Batch Watershed Delineation |
| Adjoint Catchment |  | Batch Watershed Delineation |
| Batch Point |  | Batch Watershed Delineation |
| Watershed Point | Batch Watershed Delineation |  |
| Watershed | Batch Watershed Delineation |  |
| Subwatershed Outlet |  | Batch Subwatershed Delineation |
| Subwatershed | Batch Subwatershed Delineation |  |
| Drainage Area |  | Drainage Area Centroid |
|  |  | Longest Flow Path |
| Centroid | Drainage Area Centroid |  |
| Longest Flow Path | Longest Flow Path |  |

### Batch Watershed Delineation

The Batch Watershed Delineation function (Watershed Processing menu) allows delineating watersheds in batch for points defined in the BatchPoint feature class.  Points are added to the BatchPoint feature class with the tool Batch Point Generation .Add_batch_point

The user is prompted for the following five input themes and two output themes:

| Input |  | Output |
| --- | --- | --- |
| "Flow Direction Grid" |  | "Watershed Point" Feature Class |
| "Stream Grid" |  | "Watershed" Polygon Feature Class |
| "Catchment" Polygon Feature Class |  |  |
| "Adjoint Catchment" Polygon Feature Class |  |  |
| "Batch Point" Feature Class |  |  |

Input/Output Management

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the Flow Direction Grid.  If not, the user needs to select an existing grid theme (of flow direction type) that will be tagged with the "Flow Direction Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Stream Grid", it will be used as a default for the Stream Grid.  If not, the user needs to select an existing grid theme that will be tagged with the "Stream Grid" tag at the end of the operation.

If there is a polygon feature class that has a tag "Catchment", it will be used as a default for the Catchment theme.  If not, the user needs to select an existing polygon feature class that will be tagged with the "Catchment" tag at the end of the operation.

Required Fields: GridID

If there is a polygon feature class that has a tag "Adjoint Catchment", it will be used as a default for the Adjoint Catchment theme.  If not, the user needs to select an existing polygon feature class that will be tagged with the "Adjoint Catchment" tag at the end of the operation.

Required Fields: GridID

If there is a point feature class that has a tag "Batch Point", it will be used as a default for the Batch Point theme.  If not, the user needs to select an existing point feature class that will be tagged with the "Batch Point" tag at the end of the operation.

Required Fields: Name, Batchdone, Snapon, Description

The user is prompted for two themes, used to store the result of the computation.  These two themes will be created and added to the ArcMap Table of Contents, if not already thereDelineate_on_a_point:

The first layer created is the WatershedPoint feature class ("Watershed Point" tag), which stored the location of the processed batch points.  The points are moved to the center of the cell in which they are located if no snapping is performed, otherwise they are moved to the snapped locations.  If there is a feature class name that has a tag "Watershed Point", it will be used as a default for the Watershed Point output feature class.  If not, the user needs to specify a feature class name that will be tagged with the "WatershedPoint" tag at the end of the operation.  If the output feature class name already exists, the new points will be appended in this existing feature class.

Fields created:

- DrainID – HydroID of the associated watershed.

- Name – Name retrieved from the Batch Point input layer.

- HydroID – Unique identifier of the watershed point in the Hydro database.

The second output layer is the Watershed polygons feature class ("Watershed" tag), which stores the delineated watersheds.  If there is a feature class name that has a tag "Watershed", it will be used as a default for the Watershed output polygon feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Watershed" tag at the end of the operation.  If the output feature class name already exists, the new polygons will be appended in this existing feature class.

Fields created:

- 	DrainID –  Same as HydroID, since Watershed is a drainage area.

- 	Name – Name retrieved from the Batch Point input layer.

- 	Comment – Comments retrieved from the Batch Point input layer.

- 	HydroID – Unique identifier of the watershed in the Hydro database.

#### How the function works

The tool performs the following operations:

- Prompt the user for the name of the themes to use as input/output.

- Check the value of the field "BatchDone" for each point in the input BatchPoint feature class:

O If BatchDone=1 or -1: the tool ignores this point, for which the batch watershed delineation has already been performed or attempted.

O If BatchDone=0: the tool performs the batch watershed delineation for this point.

- Add the point for which a watershed is delineated in the WatershedPoint feature class ("WatershedPoint" tag). The function checks the value of the field SnapOn in the BatchPoint feature class to perform the snapping.  If snapping is requested, all the points located within 4 cells of the stream grid will be snapped.

- Create the associated watershed in the Watershed feature class ("Watershed" tag), even for the points that could not be snapped.

- Update the value of the field BatchDone from ‘0’ to’1’ for the point in the input batch point feature class if the delineation was successful, and to ‘-1’ if the delineation failed.

### Batch Subwatershed Delineation

The Batch Subwatershed Delineation function (Watershed Processing menu) allows delineating subwatersheds for all the points in a selected Point Feature Class.Batch_Point_Generation Add_batch_point

| Input |  | Output |
| --- | --- | --- |
| "Flow Direction Grid" |  | "Subwatershed" Polygon Feature Class |
| "Subwatershed Outlet" Point Feature Class |  |  |

Input/Output Management

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the Flow Direction Grid.  If not, the user needs to select an existing grid theme (of flow direction type) that will be tagged with the "Flow Direction Grid" tag at the end of the operation.

If there is a point feature class that has a tag "Subwatershed Outlet", it will be used as a default for the point theme.  If not, the user needs to select an existing point feature class that will be tagged with the "Subwatershed Outlet" tag at the end of the operation.

If there is a feature class name that has a tag "Subwatershed", it will be used as a default for the Subwatershed output polygon feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Subwatershed" tag at the end of the operation.  If the output feature class already exists, the user is prompted whether to remove the existing dataset.

Fields created:

- GRIDCODE – OID of the corresponding point in the input Point Feature Class.

- HydroID – Unique identifier in the Hydro database.

- DrainID – Same as HydroID since Subwatershed is a drainage area.

### Drainage Area Centroid

The function Drainage Area Centroid (Watershed Processing menu) generates the centroid of drainage areas as centers of gravity.  It operates on a selected set of drainage areas in the input feature class ("DrainageArea" tag). If no drainage area has been selected, the function operates on all the drainage areas.

| Input |  | Output |
| --- | --- | --- |
| "Drainage Area" Feature Class |  | "Centroid" Feature Class |

#### Input/Output Management

If there is a polygon feature class that has a tag "DrainageArea", it will be used as a default for the input theme.  If not, the user needs to select an existing polygon feature class that will be tagged with the "DrainageArea" tag at the end of the operation.

Required field: HydroID

If there is a point feature class that has a tag "Centroid", it will be used as a default for the output feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Centroid" tag at the end of the operation.  The records in the existing output feature class will be cleared.  The "DrainageArea" tag is also attached to the output Centroid feature class: it indicates the drainage area feature class for which the centroids have been computed.

Field created: DrainID: HydroID of the drainage area corresponding to the centroid.

Note: HydroIDs can be created with the function Assign HydroID.

### Longest Flow Path

The function Longest Flow Path (Watershed Processing Menu) identifies and computes the length of the longest flow path in a selected set of drainage areas.  If no drainage area has been selected, the function processes all the drainage areas.

| Input |  | Output |
| --- | --- | --- |
| "Flow Direction Grid" |  | "Longest Flow Path" Feature Class |
| "Drainage Area" Feature Class |  |  |

#### Input/Output Management

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the first input grid.  If not, the user needs to select a grid name that will be tagged with the "Link Grid" tag at the end of the operation.

If there is a polygon feature class that has a tag "Drainage Area", it will be used as a default for the second input theme.  If not, the user needs to select an existing polygon feature class that will be tagged with the "Drainage Area" tag at the end of the operation.

Required field: HydroID

If there is a theme that has a tag "Longest Flow Path", it will be used as a default for the output line feature class.  If not, the user needs to specify a feature class name that will be tagged with the " Longest Flow Path" tag at the end of the operation.  The records in the existing output feature class will be cleared.  The "Drainage Area" tag is also attached to the output feature class: it indicates the drainage area feature class for which the longest flow path feature class was generated.

Field created: DrainID: HydroID of the drainage area for which the longest flow path was computed.

Note: HydroIDs can be created with the function Assign HydroID.

#### How the function works

The function computes the longest flow path within each drainage area and creates/updates the longest flow path theme ("Longest Flow Path " tag).  If a path already exists for that drainage area, it will be replaced with the newly defined one.

## Attribute Tools

### About Attribute Tools

The Attributes Tools menu provides access to several functions that allows reading and writing attributes in specified tables.  The following tools are available.

- Data Management: set the tags for the themes used in the Attribute Tools menu.

- Assign HydroID: assign a unique identifier (HydroID) to a feature.  HydroID is unique across a geodatabase.

- Generate From/To Node for Lines: generate from-node/to-node topology based on physical line connectivity for a line feature class (do not require hydro network).  Nodes are defined as ends of lines.  They are not created as a separate feature class, but rather just identified and accounted for internally.

- Find Next Downstream Line: find the HydroID of the next downstream linear feature class and store it in the NextDownID field of the feature.  The directionality is based on the digitized direction.  Connectivity is established by the physical connection of the linear features (do not require hydro network).

- Calculate Length Downstream for Edges: calculate length from the downstream end of a hydro edge to the outlet of the hydro network (require hydro network).  The length is stored in the LengthDown field.

- Calculate Length Downstream for Junctions: calculate length from a hydro junction to the outlet of the hydro network (require hydro network).  The length is stored in the LengthDown field.

- Find Next Downstream Junction: find the HydroID of the next downstream junction and store it in the NextDownID field of the junction feature (require hydro network).

- Store Area Outlets: identify most likely hydro junction that drains an area.  The HydroID of that junction is stored in the JunctionID field for the area feature class.

- Consolidate Attributes: consolidate the values of a numerical attribute of a feature class based on a predefined relationship, and store them in a field.  Operators include sum, min, max, average, and count.

- Accumulate Attributes: accumulate the value of a numerical attribute of a feature class based on a trace, and store them in a field.

### Data Management - Attribute Tools

Arc Hydro manages the input/output to the tools by using tags that are automatically assigned by the functions to the selected inputs and outputs.  A tag may be used as input by one function and as output by another one.  For example, the "Hydro Edge" tag is an output from Hydro Network Generation and an input to Calculate Length Downstream for Edges.

The Data Management function in the About_Basin_PreprocessingAttribute Tools menu provides a global view of the tags assignments for that menu in the active Map/Data Frame.  The function also allows assigning, reassigning or resetting the tags.  A tag may be reset by selecting "Null" as the corresponding layer.  When a reset tag is used as output, the function presents the user with default layer name associated to the tag.  This default is defined in the XML file and may be modified (see XML Manager).

#### How the function works

- The pull down menu next to each tag offers for selection only those themes in the active data frame that match the required theme type (line, point, polygon, grid).

- A "Null" option is available in the list.  Selecting "Null" remove the theme assigned to that role.

- A layer may be assigned several tags.  When a layer is used with an additional tag, the user is warned that at least one other tag has already been assigned to this theme.

- tag may be assigned to one and only layer at any time.

#### Tags defined in Attribute Tools

| Tag | Input to |
| --- | --- |
| Line | Generate From/To Node for Line |
|  | Find Next Downstream Line |
| Hydro Edge | Calculate Length Downstream for Edges |
| Hydro Junction | Calculate Length Downstream for Junctions |
|  | Store Area Outlets |
| Area | Store Area Outlets |
| Drainage Point | Store Area Outlets |

### Assign HydroID

The Assign HydroID function (Attribute Tools menu) allows assigning or reassigning a unique identifier, the HydroID, to the features in the Arc Hydro database.

The user needs to select the following parameters:

- Map/Data Frame. Default to the active Map.

- Workspace (Geodatabase).

- Layers whithin the selected map/workspace on which to operate.

- Whether to overwrite existing HydroIDs.

- Whether to operate on all features on all selected features only.

- HydroIDs are assigned based on sequences defined in two tables, HydroIDTable and LayerKeyTable.  These tables are created by default in the target workspaces, with the following structure:

- *Create_HydroID_Tables

| HydroIDTable |  | LayerKeyTable |
| --- | --- | --- |
| LayerKey |  | - LayerName |
| HydroID |  | - LayerKey. |

HydroIDs are generated in the following way:

- The LayerKey associated with the layer is retrieved from the LayerKeyTable (if no key is defined, then the key "OTHERS" is used).

- The HydroID associated to that key is retrieved from the HydroIDTable.

- The HydroID is incremented (by 1 by default) to the NextHydroId value.

- The HydroID field is created (if needed) in the layer and populated with the incremented HydroID.

- The LayerKeyTable is created empty, whereas the HydroIDTable is created with the following records:

| LayerKey |  | HydroID |
| --- | --- | --- |
| OTHERS |  | - 0 |
| LayerKey1 |  | - 1000000 |
| LayerKey2 |  | - 2000000 |
| LayerKTey3 |  | - 3000000 |

- The HydroIDs tables may be edited with the HydroID Tables Manager.

### Generate From/To Node for Lines

The function Generate From/To Node for Lines (Attribute Tools menu) updates the FROM_NODE and TO_NODE fields for each line feature in the "Line" feature class.  The function creates the fields if they do not already exist.

A node is the beginning or the ending location of a line feature.  It is topologically linked to all lines that meet at the node.  From nodes and to nodes are assigned based on the direction in which the line features were digitized.  The FROM_NODE and TO_NODE fields define both the direction in which the line features have been digitized, but also the links between the different line features.

| Input |  | Output |
| --- | --- | --- |
| "Line" Feature Class |  | FROM_NODE updated in "Line" Feature Class |
|  |  | TO_NODE updated in "Line" Feature Class |

#### Input/Output Management

If there is a line feature class that has a tag "Line", it will be used as a default for the input theme.  If not, the user needs to select an existing line theme name that will be tagged with the "Line" tag at the end of the operation.

Fields created:

- FROM_NODE

- TO_NODE

### Find Next Downstream Line

The function Find Next Downstream Line (Attribute Tools menu) finds the next downstream element in a linear feature class based on the digitized direction, and assigns the HydroID of this downstream feature to the NextDownID field in the feature class.  Having the next downstream feature listed as an attribute in the table allows features to "communicate" with each other without the presence of a network, passing values or other information as desired.  Knowing the next downstream point of interest is also important for applications such as Water Rights Analysis and Total Maximum Daily Load studies.

| Input |  | Output |
| --- | --- | --- |
| "Line" Feature Class |  | "NextDownID" field in "Line" feature class |

#### Input/Output Management

If there is a line feature class that has a tag "Line", it will be used as a default for the input theme.  If not, the user needs to specify a theme name that will be tagged with the "Line" tag at the end of the operation.

Required field: HydroID

Fields created:

- NextDownID

- FROM_NODE

- TO_NODE

#### How the function works

For each line feature in the "Line" feature class, the function creates (if needed) and populates the FROM_NODE and TO_NODE fields (see Generate From/To Node for Lines function).  Then it retrieves the TO_NODE of the feature and look for the feature having the same value for its FROM_NODE: this is the next downstream feature for the feature being processed.  The function retrieves the HydroID of the downstream feature, and populates the NextDownID of the processed feature with this value.

If the function cannot find a downstream feature, then the NextDownID is populated with –1.

#### Limitation

The function will retrieves the HydroID of the first features, whose FROM_NODE is the same as the TO_NODE.  It does not handle multiple downstream features.

### Calculate Length Downstream for Edges

The function Calculate Length Downstream for Edges (Attribute Tool menu) calculates the length from a network edge to the sink that the edge flows to, and populates the field LengthDown in that feature class with the calculated value.

The user is prompted for a network edge feature and for the Length field to use in that feature class.

| Input |  | Output |
| --- | --- | --- |
| "Hydro Edge" Complex Edge Feature Class |  | "LengthDown" Field in "Hydro Edge" |
| "Length" Field for "Hydro Edge" |  |  |

#### Input/Output Management

If there is a network edge feature class that has a tag "Hydro Edge", it will be used as a default for the input theme.  If not, the user needs to select an existing network edge theme name that will be tagged with the "Hydro Edge" tag at the end of the operation.

The user is also prompted for the field containing the length of each feature in the "Hydro Edge" feature class.  The type of this field must be Double.

Field created: LengthDown

#### How the function works

The length downstream value starts at zero at a sink and increases in the upstream direction.  The length downstream value for a given edge includes the length of all downstream edges (taking the shortest path through any loops), but not the length of the current edge.  For instance, the length downstream for an edge at the most upstream segment of a river (with no loops) will be the entire length of the river minus the length of that upstream segment.  Likewise, the length downstream for any edge that is connected to a sink will be zero.

The tool works by tracing downstream from a given feature and summing up the lengths of all downstream edges that comprise the shortest path between the feature and the nearest sink.  The length of each edge is defined in a field specified by the user.  Null values in the field are treated as zeros.  The length downstream is recorded in a field called LengthDown of type double.  If a LengthDown field does not already exist in the feature class, it will be created automatically.

The tool can run on a selected set of records or on all records.  If no features are selected, the tool runs on all records.  If any features are selected, the tool runs only on the selected records.

#### Network preparation

Before using the tool, several steps should be taken to ensure that the network and network solver are prepared to perform the length downstream calculations:

- Flow direction must be set in the network.  If no flow direction is set, then the length downstream for every feature will be zero (because the feature does not know where to flow).  To set flow direction, use the Set Flow Direction button on the Utility Network Analyst toolbar, or Set Flow Direction on the Arc Hydro Tools toolbar.

- If the network contains loops or is otherwise not a dendritic network (i.e. does not branch in the downstream direction), then weights and the Trace Indeterminate Flow option must be specified using the Utility Network Analyst.  If flow direction has not been set for edges in a loop and the network solver has not been instructed to trace through indeterminate flow, then a downstream trace will end at the beginning of the loop.

- The Trace Indeterminate Flow option is set using the Utility Network Analyst toolbar by clicking Analysis->Options and selecting the General tab.  With this option checked, a downstream trace will proceed through loops with indeterminate flow. Note that if flow direction is defined on edges in a loop, then a trace will proceed according to each edge’s flow direction.

- The tool uses the lengths of edges along the shortest path between the feature of interest and the nearest sink.  This path is determined by the network weights.  If no weights are specified, then the path that traces through the least number of edges is considered the shortest path.  This can lead to inaccurate results since the path with the least number of edges may not necessarily be the shortest path as far as distance goes.  Note that this is not a problem with dendritic networks since there is only one path to the sink in such networks.  Weights are set using the Utility Network Analyst toolbar by clicking Analysis->Options and selecting the Weights tab.  Note that all weights for the network must be defined when the network is created.  Refer to the ArcGIS Desktop Help for more information on weights and networks.

### Calculate Length Downstream for Junctions

The function Calculate Length Downstream for Junctions (Attribute Tool menu) calculates the length from a network junction to the sink that the junction flows to, and populates the field LengthDown in that feature class with the calculated value.

The user is prompted for a network junction feature and for the Length field to use in all the edge feature classes in the network in that feature class.

| Input |  | Output |
| --- | --- | --- |
| "Hydro Junction" Network Junction Feature Class |  | "LengthDown" Field in "Hydro Junction" |
| "Length" field for each network edge feature class |  |  |

#### Input/Output Management

If there is a network junction feature class that has a tag "Hydro Junction", it will be used as a default for the input theme.  If not, the user needs to select an existing network junction theme name that will be tagged with the "Hydro Junction" tag at the end of the operation.

The user is also prompted for the field containing the edge’s length in each edge feature class participating to the network.  This field must be of type double.

Field created: "LengthDown"

#### How the function works

The length downstream value starts at zero at a sink and increases in the upstream direction.  The length downstream value for a given edge includes the length of all downstream edges (taking the shortest path through any loops), but not the length of the current edge.  For instance, the length downstream for an edge at the most upstream segment of a river (with no loops) will be the entire length of the river minus the length of that upstream segment.  Likewise, the length downstream for any edge that is connected to a sink will be zero.

The tool works by tracing downstream from a given feature and summing up the lengths of all downstream edges that comprise the shortest path between the feature and the nearest sink.  The length of each edge is defined in a field specified by the user.  Null values in the field are treated as zeros.  The length downstream is recorded in a field called LengthDown of type double.  If a LengthDown field does not already exist in the feature class, it will be created automatically.

The tool can run on a selected set of records or on all records.  If no features are selected, the tool runs on all records.  If any features are selected, the tool runs only on the selected records.

#### Network preparation

Before using the tool, several steps should be taken to ensure that the network and network solver are prepared to perform the length downstream calculations:

- Flow direction must be set in the network.  If no flow direction is set, then the length downstream for every feature will be zero (because the feature does not know where to flow).  To set flow direction, use the Set Flow Direction button on the Utility Network Analyst toolbar, or Set Flow Direction on the Arc Hydro Tools toolbar.

- If the network contains loops or is otherwise not a dendritic network (i.e. does not branch in the downstream direction), then weights and the Trace Indeterminate Flow option must be specified using the Utility Network Analyst.  If flow direction has not been set for edges in a loop and the network solver has not been instructed to trace through indeterminate flow, then a downstream trace will end at the beginning of the loop.

- The Trace Indeterminate Flow option is set using the Utility Network Analyst toolbar by clicking Analysis->Options and selecting the General tab.  With this option checked, a downstream trace will proceed through loops with indeterminate flow. Note that if flow direction is defined on edges in a loop, then a trace will proceed according to each edge’s flow direction.

- The tool uses the lengths of edges along the shortest path between the feature of interest and the nearest sink.  This path is determined by the network weights.  If no weights are specified, then the path that traces through the least number of edges is considered the shortest path.  This can lead to inaccurate results since he path with the least number of edges may not necessarily be the shortest path as far as distance goes.  Note that this is not a problem with dendritic networks since there is only one path to the sink in such networks.  Weights are set using the Utility Network Analyst toolbar by clicking Analysis->Options and selecting the Weights tab.  Note that all weights for the network must be defined when the network is created.  Refer to the ArcGIS Desktop Help for more information on weights and networks.

### Find Next Downstream Junction

The function Find Next Downstream Junction (Attribute Tools menu) uses the flow direction set in the geometric network to find the next downstream junction in a network junction feature class, and assigns the HydroID of this downstream feature to the NextDownID field in the feature class.  The function operates either on a selected set of junctions, or on the entire feature class if no junctions are selected.

Having the next downstream feature listed as an attribute in the table allows features to "communicate" with each other without the presence of a network, passing values or other information as desired.  Knowing the next downstream point of interest is also important for applications such as water rights analysis and Total Maximum Daily Load studies.

| Input |  | Output |
| --- | --- | --- |
| "Hydro Junction" Network Feature Class |  | "NextDownID" field in "Hydro Junction" |

#### Input/Output Management

If there is a network junction feature class that has a tag "Hydro Junction", it will be used as a default for the input theme.  If not, the user needs to specify a theme that will be tagged with the "Hydro Junction" tag at the end of the operation.

Required fields: HydroID

Field created: NextDownID

Note: The tool requires that flow direction be already set in the network.  The option Trace Indeterminate Flow should also be set in the Network Solver.

#### How the function works

For each junction in the network, the function selects an adjacent edge that carries flow away from that junction (as defined by the flow direction of the edge).  It then finds the closest junction on that edge and checks to see if that junction is in the same feature class as the starting junction.  If so, then it identifies that junction as the next downstream junction and copies that junction’s HydroID to the NextDownID field in the starting junction.  If the junction is not in the same feature class, it may perform (upon the user’s discretion) a spatial search at that location to see if any junctions from the starting junction’s feature class are spatially coincident with the junction that was found on the edge.  If such a junction is found, it identifies it as the next downstream junction.  If no junctions are found, it continues to search edge by edge in the downstream direction.  If no next downstream junction is found by the end of the search, the starting junction is assigned a value of "-1".

Limitation

In some networks, junctions may be spatially coincident with each other.  When this happens, the network builder chooses one and only one junction to participate in the network.  If the Find Next Downstream Junction is processing a junction that is not on the network, then the tool (upon the user’s discretion) will search for another junction at the exact same location that is on the network.  The function will operate much more slowly in a network with multiple junction feature classes, since it may have to perform many spatial searches while locating the next downstream junction.  If there is only one populated junction feature class in the network, the tool will operate very quickly.

### Store Area Outlets

The function Store Area Outlets (Attribute Tools menu) locates the outlet junctions for a selected set of areas and assigns the HydroID of the junction to the JunctionID field in the corresponding area feature class.  If no features are selected, the tool runs on all records.  The JunctionID field is created if it does not already exist in the area feature class.

This tool is designed to facilitate a scheme where areas are connected to the network through outlet junctions.  To perform hydrologic analyses involving rainfall/runoff and channel routing, areas must somehow be connected to the river network in order to pass runoff to the river channel.  This connection is established by storing in the JunctionID of the area feature class field the HydroID of the junction that serves as the outlet for the area.  Hypothetically, a junction may serve as the outlet of multiple areas, but each area will have only one outlet.

The tool uses one of the three following methods for locating the outlet junction:

- Junction Intersect Method

- Drainage Point Proximity Method

- Next Downstream Area Method

If the tool cannot locate an outlet junction for a given area, the JunctionID field for that area is set to -1.  A message informs the user that the ObjectIDs of those areas have been written to an error log.  These areas may also be retrieved by selecting the areas with a –1 JunctionID value.

Note: Before using the tool, the Trace Indeterminate Flow option in the Network Solver must be set.

#### Junction Intersect Method

This method requires the user to select as input a Hydro Junction feature class and an Area feature class.

| Input |  | Output |
| --- | --- | --- |
| "Hydro Junction" Feature Class |  | JunctionID field in "Area" Feature Class |
| "Area" Polygon Feature Class |  |  |

#### Input/Output Management

If there is a network junction feature class that has a tag "Hydro Junction", it will be used as a default for the first input theme.  If not, the user needs to select an existing network junction feature class that will be tagged with the "Hydro Junction" tag at the end of the operation.

Required Field: HydroID

If there is a network junction feature class that has a tag "Area", it will be used as a default for the second input theme.  If not, the user needs to select an existing polygon feature class that will be tagged with the "Area" tag at the end of the operation.

Field created: JunctionID

#### How the Junction Intersect method works

The Junction Intersect Method selects the junctions that lie within a specified distance (search tolerance) of the boundary of the drainage area.  From these junctions, upstream and downstream traces are performed, with the results returned as a set of edges.  For each junction, the resulting edges are intersected with the area to find the total length of upstream edges in the area, and the total length of downstream edges in the area, using the given junction as a reference point.  The junction that produces the greatest (upstream length - downstream length) value is designated as the outlet junction, provided that the upstream length is greater than or equal to 5 times the downstream length.  This last criterion helps to ensure that the chosen junction is not located in the middle of the drainage area.  If no junctions are located on the exact boundary of the drainage area, the search tolerance can be increased to select junctions that are near the area’s boundary.

Note: The Junction Intersect method does not create junctions at the intersection of the areas and the network.  The tool assumes that this step has already been done.  One way to produce junctions on the boundaries of areas is to intersect the line set with the areas, using the Geoprocessing Wizard’s Intersect Utility.  Rebuilding the network from the intersected line set will produce junctions where the lines and areas intersected.

The following figures illustrate the process of selecting the outlet junction.  The tool is operating on the pink area in this example.

#### Limitation

The Junction Intersect Method possesses some limitations.  To insure that all junctions along the boundary of an area are selected, the tool builds two buffer polygons, one bigger and one smaller than the drainage area.  The user specifies the size of the buffer as the search tolerance for this method.  Careful consideration should be taken in choosing the search tolerance.  A tolerance too large may result in selecting junctions far downstream of the area.  A tolerance too small may result in not selecting the appropriate junction.  The nature of the map units should also be taken into consideration.  When working in a coordinate system where 1 map unit corresponds to a great distance, such as a geographic coordinate system, a tolerance of 50 may select a great number of potential outlet junctions.  Data should be projected to a coordinate system with map units that provide sufficient precision in measurements before running the Store Area Outlets tool.

The Junction Intersect Method works best with a relatively homogeneous set of drainage areas.  If the areas differ greatly in size, then a search tolerance that is appropriate for one area may be too large or too small for another area.

#### Drainage Point Proximity Method

This method requires the user to select as input a Hydro Junction feature class, an Area feature class and a Drainage Point feature class.

| Input |  | Output |
| --- | --- | --- |
| "Hydro Junction" Feature Class |  | JunctionID field in "Area" Feature Class |
| "Area" Polygon Feature Class |  |  |
| "Drainage Point" Feature Class |  |  |

Input/Output Management

If there is a network junction feature class that has a tag "Hydro Junction", it will be used as a default for the first input theme.  If not, the user needs to select an existing network junction feature class that will be tagged with the "Hydro Junction" tag at the end of the operation.

Required field: HydroID

If there is a network junction feature class that has a tag "Area", it will be used as a default for the second input theme.  If not, the user needs to select an existing polygon feature class that will be tagged with the "Area" tag at the end of the operation.

Required field: DrainID

Field created: JunctionID

If there is a point feature class that has a tag "Drainage Point", it will be used as a default for the third input theme.  If not, the user needs to select an existing point feature class that will be tagged with the "Drainage Point" tag at the end of the operation.

Required field: DrainID

How the Drainage Point Proximity method works

Drainage Areas are often delineated from a digital elevation model (DEM).  A Drainage Point typically represents the centroid of the outlet cell of the DEM to which all other cells within the Drainage Area flow.  A Drainage Point does not necessarily fall on the stream network, especially if that network was derived from means other than grid processing.  The Drainage Point Proximity method locates the junction nearest to the Drainage Point for a given area, within the specified search tolerance, and designates this junction as the outlet junction.  The DrainID attribute in the area feature class points to the DrainID attribute in the Drainage Point feature class.  These attributes must already exist and must be populated in both the drainage area feature class and drainage point feature class in order to use this method.  If no junctions are found within the specified search tolerance, a value of –1 is assigned to the JunctionID field in the drainage area feature class.

The figure below shows a potential scenario for use of the Drainage Point Proximity Method.

When using the DrainID method, note that if the nearest junction to the Drainage Point for a given area is not in the same drainage area as the desired outlet junction feature class, then a value of –1 is assigned as the JunctionID for that area.

Next Downstream Area Method

The Next Downstream Area Method requires one input class, the Area Feature class, where the attributes NextDownID and HydroID already exist and are populated.

| Input |  | Output |
| --- | --- | --- |
| "Area" Polygon Feature Class |  | JunctionID field in "Area" Feature Class |

Input/Output Management

If there is a network junction feature class that has a tag "Area", it will be used as a default for the input theme.  If not, the user needs to select an existing polygon feature class that will be tagged with the "Area" tag at the end of the operation.

Required fields: HydroID, NextDownID

Field created: JunctionID

How the NextDownstream Area method works

Connectivity between Drainage Areas may be defined through the NextDownID attribute.  On a Drainage Area, the NextDownID points to the HydroID of the next downstream Drainage Area.  Thus, connectivity between Drainage Areas may be established without the use of a geometric network.  The Next Downstream Area method uses this connectivity to locate the outlet junction for a Drainage Area.  For a given area, this method reads JunctionID values from next downstream areas until a numeric non-zero value (that is not equal to –1) is found.  This value is then assigned as the JunctionID for the area of interest.  This technique is useful when no stream network extends into the area of interest.

The following figure illustrates the Next Downstream Area method.  The area in pink is the area of interest in this case.  The tool looks for an area with a HydroID equal to the NextDownID of the pink area (101).  The next downstream area is the area at the bottom of the figure (HydroID = 101).  The JunctionID is read from this downstream area and recorded in the pink area, so that both areas share the same outlet.

### Consolidate Attributes

The Consolidate Attributes tool (Attribute Tools menu) allows consolidating the source attribute in the source layer based on a relationship between the source layer and the target layer.  Only layers having relationships may be selected as target or source layer.  The source has to be different from the target, and related to it.

How the function works

For each target feature, the function finds the related source features based on a predefined relationship, and performs the selected consolidation operation on these source features.  The result of the consolidation is stored in the target field.  Options for consolidation operations include Sum, Max, Min, Count, Average, Median, Mode, and Standard Deviation.

### Accumulate Attributes

The function Accumulate Attributes (Attribute Tools menu) allows accumulating attributes of target features located upstream of source features.  Target features may either belong to the source feature class, or to a layer related to the source feature class.  Upstream target features are located by performing a trace on the target feature class or on a related feature class.  Two types of trace may be used: based on a geometric network; based on the NextDownID attribute.

#### How the function works

The function operates on a selected set of features or on all features.  It traces from each feature to locate all the related features located upstream.  The accumulation may be performed in the source feature class or in any layer related through a predefined relationship class

The following parameters need to be defined:

- Trace Type: Geometric Network trace or NextDownID trace.

- Source layer: layer containing the field that will be accumulated.  This layer has to be the layer on which the trace is being performed, or a layer related to that layer through a relationship. Must belong to a geometric network for a Geometric Network trace.

- Source field: field to accumulate.

- Accumulation Type: operations include sum, average, min, max, count, median, mode, standard deviation, weighted average.

- Target layer for Accumulation: same as source layer or related layer.

- Target field: field used to stored the result of the accumulation.  The field may be created if the target layer does not contain selected features.

## Network Tools

### About Network Tools

The Network Tools menu provides access to several functions that allows processing the hydrologic network.  The following tools are available.

- Data Management: set the tags for the themes used in the Network Tools menu.

- Hydro Network Generation: generate a hydro network (hydro edges and hydro junctions) from drainage lines, catchments, and drainage points.  The function updates all the connectivity fields in input feature classes.

*Hydro_Network_Generation

- Node/Link Schema Generation: generate schematic (node-link) network by connecting centers of catchments/drainage areas and junctions, and junctions.  Connectivity is established through connectivity fields (attributes), not physical connectivity.

- Store Flow Direction: store information about hydro (geometric) network element’s directionality into an attribute of the feature matching the element.

- Set Flow Direction: define flow direction for a geometric network based on digitized direction or an attribute for the feature.

### Data Management - Network Tools

Arc Hydro manages the input/output to the tools by using tags that are automatically assigned by the functions to the selected inputs and outputs.  A tag may be used as input by one function and as output by another one.  For example, the "Hydro Edge" tag is an output from Hydro Network Generation and an input to Store Flow Direction.

The Data Management function in the About_Basin_PreprocessingNetwork Tools menu provides a global view of the tags assignments for that menu in the active Map/Data Frame.  The function also allows assigning, reassigning or resetting the tags.  A tag may be reset by selecting "Null" as the corresponding layer.  When a reset tag is used as output, the function presents the user with default layer name associated to the tag.  This default is defined in the XML file and may be modified (see XML Manager).

#### How the function works

- The pull down menu next to each tag offers for selection only those themes in the active data frame that match the required theme type (line, point, polygon, grid).

- A "Null" option is available in the list.  Selecting "Null" remove the theme assigned to that role.

- A layer may be assigned several tags.  When a layer is used with an additional tag, the user is warned that at least one other tag has already been assigned to this theme.

- tag may be assigned to one and only layer at any time.

#### Tags defined in Network Tools

| Tag | Output from | Input to |
| --- | --- | --- |
| Drainage Line |  | Hydro Network Generation |
| Catchment |  | Hydro Network Generation |
| Drainage Point |  | Hydro Network Generation |
| Hydro Edge | Hydro Network Generation | Store Flow Direction |
|  |  | Set Flow Direction |
| Hydro Junction | Hydro Network Generation |  |
| Watershed Polygons |  | Node/Link Schema Generation |
| Junctions |  | Node/Link Schema Generation |
| Schema Link | Node/Link Schema Generation |  |
| Schema Node | Node/Link Schema Generation |  |

### Hydro Network Generation

The Hydro Network Generation function (Network Tools menu) allows converting drainage features into network features.  The function performs the following operations:

- Convert the Drainage Line feature class into the Hydro Edge feature class.

- Create the Hydro Junction feature class as the From Nodes/To Nodes without duplication (a junction is defined by one and one point only)

- Create an Arc Hydro Geometric Network based on the Hydro Edge and Hydro Junction feature classes, and set the flow direction in Hydro Edge in the digitized direction.

- Establish attribute connectivity between the drainage and network feature classes.

- Attribute each junction with HydroID and NextDownID field.

- Create a relationship between the Catchments and Hydro Junctions: HydroJunctionHasCatchment

| Input |  | Output |
| --- | --- | --- |
| "Drainage Line" Feature Class |  | "Hydro Junction" Network Junction Feature Class |
| "Catchment" Polygon Feature Class |  | "Hydro Edge" Complex Edge Feature Class |
| " Drainage Point" Feature Class |  | "Arc Hydro" Geometric Network |
|  |  | "HydroJunctionHasCatchment" Relationship |

#### Input/Output Management

If there is a theme that has a tag "Drainage Line", it will be used as a default for the first input feature class.  If not, the user needs to select a line feature class that will be tagged with the "Drainage Line" tag at the end of the operation.

Required fields: GridID, HydroID, NextDownID

Fields created/updated: DrainID

If there is a theme that has a tag "Catchment", it will be used as a default for the second input feature class.  If not, the user needs to select a polygon feature class that will be tagged with the "Catchment" tag at the end of the operation.

Required fields: GridID, HydroID, DrainID

Fields created/updated: NextDownID, JunctionID

If there is a theme that has a tag "Drainage Point", it will be used as a default for the third input feature class.  If not, the user needs to select a point feature class that will be tagged with the "Drainage Point" tag at the end of the operation.

Required fields: GridID

Fields created: HydroID (if not previously existing). Created empty. May be populated by Assign HydroID.

Fields created/updated: NextDownID, DrainID, JunctionID

If there is a theme that has a tag "Hydro Edge", it will be used as a default for the output complex edge feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Hydro Edge" tag at the end of the operation.  The output feature class, if it already exists, is automatically overwritten without prompting the user.

Fields created:

- HydroID: Unique identifier of the Hydro Edge in the Hydro database.

- ReachCode: Reach code.  Corresponds to the DrainID in the Drainage Line feature class, referring to the DrainID of the catchment where the reach is located.

- FlowDir: Flow direction for the edge. Defaults to WithDigitized flow direction.

- FType: Feature Type.  Defaults to Synthetic Channel.

- EdgeType: Type of Edge. Available values are Flowline and Shoreline.  Default to Flowline.

- Enabled: Indicates whether the feature is enabled (True) or disabled (False) in the geometric network.  A feature that is disabled in the network acts as a barrier when the network is traced.

If there is a theme that has a tag "Hydro Junction", it will be used as a default for the output network junction feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Hydro Junction" tag at the end of the operation.  The output feature class, if it already exists, is automatically overwritten without prompting the user.

Fields created:

- 		*Hydro_Network_GenerationHydroID: Unique identifier of the Hydro Junction in the Hydro database.

- NextDownID: HydroID of the next downstream Hydro Junction.

- FType: Feature type of the Hydro Junction: Stream Confluence, Drainage Outlet, Null for beginning of streams (for the entire drainage area considered)

- Enabled: Indicate whether the feature is enabled (True) or disabled (False) in the geometric network.  A feature that is disabled in the network acts as a barrier when the network is traced.

- AncillaryRole: Specify whether the feature represents a source, sink or neither.  Used to calculate the flow direction in the geometric network (1: Source, 2: Sink (Drainage Outlet), 0: None).

#### Attribute Connectivity

Hydro Network Generation establishes connectivities between the input and output feature classes through the JunctionID and NextDownID attributes.

- JunctionID is the HydroID of the Hydro Junction related to the feature (Catchment or Drainage Point). There is one and only one JunctionID for a given catchment or drainage point, but there may be several catchments or drainage points associated to a Hydro Junction.

- NextDownID is the HydroID of the next down feature in the Catchment and Drainage Point feature classes.

- The relationship HydroJunctionHasCatchment relates the Hydro Junction feature class to the Cachment feature class based on the Junction HydroID/Catchment JunctionID.

### Node/Link Schema Generation

The Node/Link Schema Generation tool (Network Tools menu) allows generating a node-link schema.  The nodes are defined by the centers of the polygons representing basins and by points that represent locations of interest in the model.  The points include basin outlets, river junctions, water intakes and other facilities.

The function requires that the relationship between the Watershed Polygons and their outlet be established through the JunctionID field, and the relationship between the Junctions and their next downstream junction be established through the NextDownID field.

| Input |  | Output |
| --- | --- | --- |
| "Watershed Polygons" Feature Class (optional) |  | - "Schema Link" Line Feature Class |
| "Junctions" Point Feature Class |  | - "Schema Node" Point Feature Class |

#### Input/Output Management

If there is a polygon feature class that has a tag "Watershed Polygons", it will be used as a default for the first input theme.  If not, the user needs to select an existing polygon feature class that will be tagged with the " Watershed Polygons " tag at the end of the operation.  This input is optional.

Required fields:

- HydroID

- JunctionID: HydroID of the junction that serves as the outlet for the drainage area. May be populated by the function Store Area Outlets.

If there is a point feature class that has a tag "Junctions", it will be used as a default for the second input theme.  If not, the user needs to select an existing point feature class that will be tagged with the " Junctions " tag at the end of the operation.

Required fields:

- HydroID

- NextDownID: HydroID of the next downstream junction. May be populated by the function Hydro Network Generation for a network junction feature class.

If there is a linear feature class that has a tag "Schema Link", it will be used as a default for the output feature class.  If not, the user needs to specify a feature class name that will be tagged with the " Schema Link " tag at the end of the operation.

Fields created:

- HydroID – Unique identifier of the Schema Link in the Hydro database.

- FromNodeID – HydroID of the Schema Node at the upstream end of the Schema Link.

- ToNodeID – HydroID of the Schema Node at the downstream end of the Schema Link.

- LinkType – Type of link.  1 for Watershed link (between Watershed and junction Schema Node), 2 for Junction link (between two junction Schema Nodes).

If there is a point feature class that has a tag "Schema Node", it will be used as a default for the output feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Schema Node " tag at the end of the operation

Fields created:

- HydroID: Unique identifier of the Schema Node in the Hydro database.

- FeatureID: HydroID of the Hydro Feature from which the node was created.  This enables the schematic node to connect to the attribute information of the feature it describes.  If necessary, a relationship can be built using these attributes

- SrcType: Source Type, i.e. input layer used to create the node (1 for Watershed "Watershed Polygons", 2 for Junction "Junctions").

#### How the function works

The function creates one Schema Node for each feature in the two input feature classes:

- For Watershed Polygons: the Schema Node corresponds to the center of gravity of the watershed.

- For Junctions: the Schema Node corresponds to the junctions.

The function creates Schema Links as:

- Watershed link, between a watershed Schema Node and its corresponding junction Schema Node outlet.  The outlet is determined by the field JunctionID in the Watershed Polygons feature class, which indicates the HydroID of the outlet in the Junctions feature class.

- Junction link, between a junction Schema Node and the next downstream junction Schema Node. The next downstream junction is retrieved from the NextDownID field in the Junctions feature class.

### Store Flow Direction

The function Store Flow Direction (Network Tools menu) reads the flow direction for a set of edges from the network and writes the value of the flow direction to a FlowDir field defined in the XML in the Edge Feature Class.

| Input |  | Output |
| --- | --- | --- |
| "HydroEdge" Complex Edge Feature Class |  | FlowDir field in Hydro Edge Feature Class |

Flow direction in a network is stored as esriFlowDirection constants.  The constants have the following values:

- esriFDIndeterminate	3	Indeterminate flow direction

- esriFDWithFlow	1	In the direction of digitization

- esriFDAgainstFlow	2	Opposite the direction of digitization

- esriFDUninitialized	0	Uninitialized

The Store Flow Direction tool reads edge flow directions from the network and writes the esriFlowDirection values to the FlowDir field in the edge feature class.  The tool creates the FlowDir field if it does not exist.

The tool can run on a selected set of records or on all records.  If no features are selected, the tool runs on all records.  If any features are selected, the tool runs only on the selected records.

Note: Make sure the flow direction has already been set for the network.

Limitations

A complex edge may have different flow directions assigned on its individual segments.  Because rows in a table represent an entire feature, and not each individual network element, the Store Flow Direction tool only stores one of the flow direction values for complex edges.  The choice of which segment to use is arbitrary.

### Set Flow Direction

The function Set Flow Direction (Network Tools menu) sets the flow direction for selected edges in a network edge feature class.  If no features are selected, the tool sets the flow direction for all the edges in the feature class.

| Input |  | Output |
| --- | --- | --- |
| "HydroEdge" Complex Edge Feature Class |  |  |

Flow direction in a network is stored as esriFlowDirection constants, which have the following values:

- esriFDIndeterminate	3	Indeterminate flow direction

- esriFDWithFlow		1	In the direction of digitization

- esriFDAgainstFlow	2	Opposite the direction of digitization

- esriFDUninitialized 	0	Uninitialized

The flow direction may be set in two ways:

- by selecting an esriFlowDirection flow direction to assign to all the selected edges (or all the edges if none selected)

- by reading values from an integer field in the edge feature class: the value must corresponds to one of the to esriFlowDirection constants, i.e. 0, 1, 2, or 3.  If not the flow direction defaults to the selected esriFlowDirection.

Assigning flow direction based on an attribute allows the user to delete the network (for maintenance or distribution reasons) and to still retain the proper flow direction values.  Once the network is reestablished, flow direction can be assigned by attribute without having to create sinks in the network.  Flow directions for indeterminate cases (such as edges in loops) can be assigned manually with this tool.  Situations in which flow directions may change (such as in canals in flat areas) can also be modeled.

Note: Complex edges may contain more than one flow direction if they are made up of more than one edge segment.  The Set Flow Direction tool assigns the selected flow direction to all segments of the complex edge.  The tool does not allow the user to set flow direction on only one individual segment of a complex edge, if that edge contains more than one segment.

## ApUtilities

### About ApUtilities

The ApUtilities menu provides access to several functions that allows configuring the map document and accessing the Arc Hydro online help.  The following functions are available.

- List Maps: list the maps/data frames present in the map document, with their associated tag.

- Add New Map: create a new map/data frame in the map document.  This function allows creating maps/data frames with spaces in the their name, that can be managed by Arc Hydro (Data Frames with spaces created with ArcMap Insert>Data Frame are not properly managed by Arc Hydro).

- Set Target Locations: set the output locations for the raster data, vector data and time series tables created by the Arc Hydro tools.

- XML Manager: manage the XML associated to the map document.

- Load/Merge Config XML: load and merge a XML file with the XML associated to the map document.

- HydroID Tables Manager: create/manage the tables managing the HydroIDs.

- Arc Hydro Help: access the Arc Hydro online help.

### List Maps

The List Maps function (ApUtilities menu) lists all the maps/data frames present in the ArcMap Table of Contents, together with their associated tag name as follows:

- MapName:name of the data frame as it appears in the Table of Contents.

- TagName: tag associated to the data frame. Cannot contain a space.

Note: The Arc Hydro tools are working on the active data frame in the Map Document.  Data Frames created through the standard ArcMap Insert>Data Frame are not managed properly by Arc Hydro when their name contain space(s).

### Add New Map

The Add New Map function (ApUtilities menu) allows adding a new Data Frame in the ArcMap Table of Contents.

Warning: Data Frame may also be created using the ArcMap function Insert>Data Frame.  However, in that case, the name of the Data Frame cannot contain space(s).

#### What is a Map in Arc Hydro?

A Map is a synonym for a Data Frame. The only difference is that Arc Hydro cannot manage a Data Frame whose name contains space(s), whereas it does manage this type of Map.

- A Data Frame is a frame on the map that displays layers occupying the same geographic area.  There may be one or more data frames in a map document depending on how the data are organized.  For instance, one data frame might provide an overview of a study area and another may highlight a specific zone of interest within the study area.

- The Arc Hydro tools operate at the map level, i.e. the tools operate only on the layers belonging to the active Data Frame.

- The tags for the layers are defined at the data frame level in the XML, under the node associated to the data frame.

- The list of maps with their associated tag can be displayed with the function List Maps.

### Set Target Locations

The Set Target Locations function (ApUtilities menu) allows assigning default target workspaces for the output data generated.  Three locations can be set for each set of tools (Arc Hydro being one set of tools):

- Raster data: the workspace may be a directory or a SDE workspace.  Default is the directory where the map document (mxd) is saved.

- Vector data: the workspace may be a directory, a personal geodatabase (Access mdb file), or a SDE database.  Default is a personal geodatabase named after the map document (e.g. if the map document is named mapdoc.mxd, then the personal geodatabase is named mapdoc.mdb), and located in the same directory as the map document.

- Time Series: the workspace may be a directory, a personal geodatabase, or a SDE database.  Default is a personal geodatabase named after the map document, and located in the same directory as the map document.

#### How the function works

- Click on ApUtilities>Set Target Location…

- Select a node.  A node is associated to set of functions.  There is always a default node (DefaultConfig).  The node corresponding to the Arc Hydro tools is HydroConfig.  Additional nodes may be defined for other sets of tools built on the same framework.

- Select the target locations for raster data, vector data and time series.  Default locations are selected when entering the window.

### XML Manager

The XML Manager (ApUtilities menu) allows managing the XML associated to the ArcMap project.  It contains the following functions:

- Browse: browse to the location of an XML.  This does not load the XML.

- Load: load the XML into the map document XML under a new secondary node (Note: the XML is not updated if the node already exists).

- Export: save the XML associated to the document to file.

- Print: print the XML

The XML manager allows performing editing in the XML associated to the map document via a right-click context menu.

The node corresponding to the Arc Hydro tools is the HydroConfig node: it contains itself four subnodes:

- TemplateView:  Default layers used by Arc Hydro.

- ProgParams: Parameters common to all the Arc Hydro functions.

- LayerKeys: Default layer keys created in the HydroID table LayerKeyTable.

- MapViews: Correspond to the maps (data frames) created in the map document.  Store the layers used in each map/data frame.

#### Note

Changes in the XML are immediately applied to the current map document.  However, the edits to the XML are permanently saved only when the map document itself is saved.  Edits may be cancelled by exiting ArcMap without saving.

### Load/Merge Config XML

The Load/Merge Config XML function (ApUtilities menu) allows loading a XML file as the XML associated to the ArcMap project, or merging an XML file with the existing XML.

The XML is loaded into the map document XML under a new secondary node (Note: the XML is not updated if the node already exists).

### HydroID Tables Manager

The HydroID Tables Manager function (ApUtilities menu) allows editing, in the selected workspace, the HydroID Tables used to generate HydroIDs.  These tables, HydroIDTable and LayerKeyTable, are automatically created in the target workspace defined for the vector data.  They have the following structure:

*Create_HydroID_Tables

| HydroIDTable |  | LayerKeyTable |
| --- | --- | --- |
| LayerKey |  | - LayerName |
| HydroID |  | - LayerKey. |

HydroIDs are generated in the following way:

- The LayerKey associated with the layer is retrieved from the LayerKeyTable (if no key is defined, then the key "OTHERS" is used).

- The HydroID associated to that key is retrieved from the HydroIDTable.

- The HydroID is incremented (by 1 by default) to the NextHydroId value.

- The HydroID field is created (if needed) in the layer and populated with the incremented HydroID.

- The HydroID Tables Manager allows editing the tables via a right-click context menu.  The function also allows prefixing the HydroIDs with a regional ID.

### Arc Hydro Help

The Help function (ApUtilities menu) allows accessing the Arc Hydro on-line Help.

## Buttons and Tools

### About Buttons and Tools

The following tools are available:

Flow Path Tracing: trace the downstream path, based on the steepest descent, from a user specified point to the edge of the DEM (use flow direction grid).

Point Delineation: delineate a watershed for a user specified point (interactive) based on the preprocessed DEM.

Batch Point Generation: add a user specified point to a batch point feature class.  This point feature class can be used as an input to the batch watershed and subwatershed delineation functions.

Assign Related Identifier: interactively assign a value of a field in a source feature to a field in the target feature.  User specifies both the source and target feature classes and fields.

Global Point Delineation: delineate a global watershed for a point located in a given Catalog Unit, by merging a watershed delineated locally in the Catalog Unit with the watersheds located in upstream Catalog Units.

Trace By NextDownID Attribute: performs a trace based on the NextDownID attribute.

### Flow Path Tracing

The Flow Path Tracing tool generates the flow path from a user-specified point.  It uses the Flow Direction Grid to determine how the water is flowing through the landscape from the selected starting point.

| Input |  | Output |
| --- | --- | --- |
| "Flow Direction Grid" |  | Flow Path Graphic |

#### Input/Output Management

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the input grid.  If not, the user needs to select an existing grid theme (of flow direction type) that will be tagged with the "Flow Direction Grid" tag at the end of the operation.

Note: To remove the result of the trace, select the trace as a graphic and click delete.

### Point Delineation

The Point Delineation tool allows creating a point and delineating the associated watershed.

The tool requires four input themes and creates/updates two themes.

| Input |  | Output |
| --- | --- | --- |
| "Flow Direction Grid" |  | "Watershed Point" Feature Class |
| "Stream Grid" |  | "Watershed" Polygon Feature Class |
| "Catchment" Polygon Feature Class |  |  |
| "Adjoint Catchment" Polygon Feature Class |  |  |

Input/Output Management

If there is a grid theme that has a tag "Flow Direction Grid", it will be used as a default for the first input grid.  If not, the user needs to select an existing grid theme name that will be tagged with the "Flow Direction Grid" tag at the end of the operation.

If there is a grid theme that has a tag "Stream Grid", it will be used as a default for the second input grid.  If not, the user needs to select an existing grid theme name that will be tagged with the "Stream Grid" tag at the end of the operation.

If there is a polygon feature class that has a tag "Catchment", it will be used as a default for the input feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Catchment" tag at the end of the operation.

Required fields: DrainID, GridID

If there is a polygon feature class that has a tag "Adjoint Catchment", it will be used as a default for the input feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Adjoint Catchment" tag at the end of the operation.

Required fields: GridID

If there is a theme that has a tag "Watershed Point", it will be used as a default for the output point feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Watershed Point" tag at the end of the operation.

The Watershed Point feature class stores the points created by the user.  This theme has the following structure:

- DrainID: HydroID of the associated Watershed.

- Name

- HydroID: Unique identifier of the watershed point in the Arc Hydro database.

If there is a theme that has a tag "Watershed", it will be used as a default for the output polygon feature class.  If not, the user needs to specify a feature class name that will be tagged with the "Watershed" tag at the end of the operation.

- The Watershed feature class contains the watersheds that have been delineated.  This theme has the following structure:

- DrainID: Same as HydroID.

- Name: Name of the point.

- Comment: Comments.

- HydroID: Unique identifier of the watershed in the Arc Hydro database.

### Batch Point Generation

The Batch Point Generation tool allows creating a Batch Point feature class, which may be used by the function Batch Watershed Delineation.

| Input |  | Output |
| --- | --- | --- |
| "Batch Point" Feature Class (requested only if the "Batch Point" tag is not assigned) |  | "Batch Point" Feature Class (optional: created if needed) New Batch Point(s) in the "Batch Point" Feature Class |

Input/Output Management

If there is a point feature class that has a tag "Batch Point", it will be used as a default for the point feature class.  Note that the function prompts for this feature class only if the "Batch Point" tag is not set.  The tag may be reset by removing the current "Batch Point" layer from the Table of Contents or by using the Data Management function in the Watershed Processing menu.

There are no required fields in the Batch Point layer: the following fields will be automatically created if they do not already exist:

- Name: Name of the point, user-entered.

- Description: Description of the point, user-entered.

- BatchDone: Indicates whether the Batch Watershed Delineation should perform a delineation on this point (‘0’: delineate, ‘1’: do not delineate).

- SnapOn: Indicates whether the point is to be snapped when performing the Batch Watershed Delineation.

- HydroID: Unique identifier of the Batch Point in the Arc Hydro database.

#### How the function works

The function first checks whether the "Batch Point" tag is assigned to a layer.  If the tag is not set, the function prompts for the name of the point feature class to use to store the batch points: this name may refer to an existing layer or to a new layer.  When the user clicks OK, the "Batch Point" feature class is added to the ArcMap Table of Contents if it is not already there.

The user can then add points to the Batch Point feature class by clicking on map at the desired location for each point, and entering the following information:

- Name (optional)

- Description (optional)

- BatchDone: default value is "0".  The Batch Watershed Delineation function will perform a delineation on this point.

- SnapOn: default value is "1", i.e. the Batch Watershed Delineation will try to snap the point.Batch_Watershed_Delineation

### Assign Related Identifier

The Assign Related Identifier tool allows updating an attribute for a target feature with the value of a related field in the corresponding source feature.

| Input |  | Output |
| --- | --- | --- |
| Source Layer/Source Field |  | Updated Target Field in selected Target feature |
| Target Layer/Target Field |  |  |

#### How the function works

- Source/Target configuration

- Click on Assign Related Identifier.

- Select the Source layer/field to use in the form.

- Select the Target layer/field (Note: HydroID cannot be the target field) in the form.

- Click OK.

- Attribute update

- On the map, select the target feature to update (Only one feature may be selected at the time.)

- Right-click the corresponding source feature, and click Assign Attribute.

The Assign Related Identifier function updates the target attribute in the selected target feature with the value of the related source attribute in the right-clicked source feature.

### Global Point Delineation

The Global Point Delineation function allows delineating the global watershed for a point located in a given Catalog Unit.  The function performs first a local delineation in the point’s Catalog Unit. It then merges the resulting local watershed with the Catalog Units polygons located upstream.

| Input |  | Output |
| --- | --- | --- |
| "Catalog Unit Junction" Network Class |  | "Global Watershed Point" |
| "Catalog Unit Edge" Network Class |  | "Global Watershed" |
| "Catalog Unit Polygon" Feature Class |  |  |

#### Input/Output Management

If there is a network junction feature class that has a tag "Catalog Unit Junction", it will be used as a default for the input theme.  If not, the user needs to select an existing network junction theme name that will be tagged with the "Catalog Unit Junction" at the end of the operation.

If there is a network edge feature class that has a tag "Catalog Unit Edge", it will be used as a default for the input theme.  If not, the user needs to select an existing network junction theme name that will be tagged with the "Catalog Unit Edge" at the end of the operation.

If there is a polygon feature class that has a tag "Catalog Unit Polygon", it will be used as a default for the input theme.  If not, the user needs to select an existing network junction theme name that will be tagged with the "Catalog Unit Polygon" at the end of the operation.

Required field: Name

If there is a theme that has a tag " Global Watershed Point*Global_Point_Delineation ", it will be used as a default for the output point feature class.  If not, the user needs to specify a feature class name that will be tagged with the " Global Watershed Point*Global_Point_Delineation " tag at the end of the operation.

The Global Watershed Point feature class stores the points created by the user.  This theme has the following structure:

- DrainID – HydroID of the drainage area (GlobalWatershed) linked to the feature.

- Name – Name entered by the user during the delineation

- HydroID – Unique identifier of the Global Watershed Point in the Arc Hydro database.

If there is a theme that has a tag " Global Watershed*Global_Point_Delineation ", it will be used as a default for the output polygon feature class.  If not, the user needs to specify a feature class name that will be tagged with the " Global Watershed*Global_Point_Delineation" tag at the end of the operation.

This theme has the following structure:

- DrainID – Unique identifier  (HydroID) of the drainage area linked to the feature.  In this particular case, GlobalWatershed being a drainage area, same as HydroID.

- Name – Name entered by the user during the delineation.

- Comment – Comments about the global watershed.

- HydroID – Unique identifier of the GlobalWatershed in the Arc Hydro database.

Prerequisites

One and only one relationship (e.g. HUCHasJunction) must exist between the Catalog Unit Junction and the Catalog Unit Polygon Feature class. The name of the relationship must match the name defined in the XML file.

Data must be stored in the Unit Catalog folder (refer to the document on Global Point Delineation), and must following the naming convention set in the XML (for Flow Direction Grid, Stream Grid, Catchment, AdjointCatchment, Network Name).  Refer to the document Global Delineation with EDNA Data for additional information on data requirements.

How the function works

- Click the on the Global Point Delineation tool.

- Click on the network edge on the point where the global delineation will be performed (if the point is not on the network, a local delineation will be performed).

- Retrieve the Catalog Unit Polygon in which the point is located, which is stored in the "Name" field.

- Retrieve the data path defined in the XML under "FrameworkConfig/HydroConfig" "/ProgParams/ApFunctions/ApFunction(GlobalDelineation)".  Data Path is the root directory under which the directories corresponding to each catalog unit are located.  These directories contain the input layers needed for the delineation.  The name of the input layers in each directory must be the same, and the layer names set in the XML must match these names.  These inputs are Flow Direction Grid, Stream Grid, Catchment, Adjoint Catchment, NetworkName and Relationship Name.  The XML may be edited using the XML Manager.

- Perform a local delineation whithin the catalog unit polygon for the point.

- Place a flag at the location of the point, and trace upstream to select the junctions located upstream in the network.

- Retrieve all the Catalog Unit Polygons corresponding to the selected junctions, based on the relationship between the polygons and junctions: the JunctionID in the polygon relates to the HydroID field in the Junction feature class.

- Merge all the selected polygons together with the locally delineated watershed, and save the resulting polygon in the Global Watershed feature class.

### Trace By NextDownID Attribute

The Trace By NextDownID Attribute function allows performing a trace on a feature class based on the NextDownID attribute.  Only layers having the attribute "NextDownID" may be traced.  The trace may be performed upstream, downstream or in both directions.

The function allows displaying the features related to the result of the trace.  It may be used for example to display the catchments located upstream and/or downstream from a specific junction.

| Input |  | Output |
| --- | --- | --- |
| Layer containing the field "NextDownID" |  | Selection containing features from the input layer located by the trace and/or related features |

#### How the function works

- Select the layer to trace (only the layers containing the field NextDownID are available for selection).

- Select whether to perform a trace upstream, downstream or in both directions.

- Select the related layers to display as a result of the trace.

- Select whether to display all the layers (including the layer used for the trace) or only the selected related layers.

- Click OK.

- Click on the map on a feature from the input layer to trace.

- The function returns the result of the trace and the related features as a selection.